2015 Tri-State

NMRA Post #6, June 2, 2015

BG-4 Written Exam Answers

Name _________________ Company ____________________

Team Name______________________ Working Order No. _____

- During the exhalation valve test, if valve is operating properly, -10 mbar is indicated on the pressure gauge.  (S.F. # 8)

- -10 mbar

- +10 mbar

- 10 mbar

- The EPDM breathing hoses use quarter turn connectors. (S.F.# 9)

- breathing hoses

- corrugated hoses

- black hoses

- The factory filled cartridge is good for 4 years from the manufacture date. (S.F. # 13)

- 4 months

- 4 years

- 5 years

- A steel cylinder is full at 3135 psi when a + is stamped at hydro test. (S.F. # 17)

- 3135 psi

- 2650 psi

- 3000 psi

- The minimum valve provides greater than 80 L/min flow. (S.F.# 30)

- relief valve

- minimum valve

- pressure valve

- The breathing bag has a 5.5 liter volume. (S.F. # 31)

- 6.6 liter

- 5.5 liter

- 6.0 liter

- The pressure reducer must be rebuilt/overhauled every 6 years. (S.F. # 34)

- 5 years

- 6 years

- 1 year

- The last low pressure warning occurs when pressure has dropped to approximately 145 psi. (S.F. # 43)

- 130 psi

- 140 psi

- 145 psi

- To start the main alarm, press the Yellow panic button in center of the Sentinel. (S.F.#46)

- Tally key

- Bypass button

- Yellow panic

- Remove the Tally Key to activate the motion sensor. (S.F. # 51)

- Panic Key

- Tally Key

- Tally Button

- Disinfect parts by immersing them in a disinfectant bath using Airkem 33. (S.F.# 56)

- AirKem 33

- Disinfect 33

- Soapy water

- The inhalation valve should be replaced every 3 years for units which are in constant use. (S.F. #65)

- 3 months

- 3 years

- 5 years

- A steel oxygen cylinder must be retested by a certified testing facility every 5 years.

- 10 years									(S.F. #69)

- 15 years

- 5 years

- It is safe to use the BG 4 for up to 4 hours with a battery warning 1 Icon. (S.F. # 76)

- 2 hours

- 4 hours

- 8 hours

- When the Tally Key is removed, the motion sensor is activated. (S.F. #80)

- motion detector

- motion sensor

- motion devise

- The oxygen cylinder safety burst disc ruptures at 4,000 psi (275 bar). (S.F. # 83)

- 3000 psi

- 3150 psi

- 4000 psi

- Polycarbonate or Plexiglas lenses can be used in the mask.  (S.F. # 86)

- Polycarbonate, Plexiglas

- Plastic, Plexiglas

- Plastic, Polycarbonate

- The drainage valve opens at approximately 15 mbar and is therefore out of the RZ reading range. (S.F. # 88)

- 10 mbar

- 15 mbar

- 10 lpm

- The breathing hoses are equipped with anti-crush rings.  (S.F. # 95)

- air hoses

- oxygen hoses

- breathing hoses

- When conducting a component check use a test pressure between ___mbar and + 10 mbar with a max pressure loss of 1 mbar/ min. (SF #98)

- – 7 mbar

- + 7 mbar

- + 6 mbar

BG-4

Paraorama Nova Mask

Pressure Reducer BG4

Minimum Valve

Cooler

Shoulder Pad Assembly

- Identify Item No. 3			A. Shoulder harness

B. Shoulder Pad

C. Shoulder Pad Assembly  (page #19)

22. Identify Item No. 5		A. Ice Chest

B. Ice Cooler

C. Cooler    		(page # 19)

23. Identify Item No. 7		A. Maximum Valve

B. Minimum Valve

C. Oxygen Valve	(page # 19)

24. Identify Item No. 17		A. High Pressure Reducer

B. Low pressure Reducer

C. Pressure Reducer BG4	(page # 19)

25. Identify Item No. 22		A. Panorama Mask

B. Paranormal Mask

C. Panorama Nova Mask	(page #19)

Cover

Button

Grip Cap

26. Identify Item No. 3		A. Button Cap

B. Grip Cap

C. Top Latch		(page # 20)

27. Identify Item No. 4		A. Button

B. Catch

C. Anchor		(page # 20)

Breathing Hose Assembly

Inhalation Valve Seat

Valve Disc

Hose Holder

28. Identify Item No. 2			A. Inhalation Valve Assembly

B. Inhalation Valve Seat

C. Inhalation Disc		(page #22)

29. Identify Item No. 4 			A. Valve Seat

B. Valve

C. Valve Disc			(page # 22)

30. Identify Item No. 7			A. Breathing Hose Holder

B. Crush Ring

C. Hose Holder 		(page # 22)