2009 Idaho Springs, Colorado Mine Rescue Contest

# Biomarine 240R Written Bench Test

Part 1    True  or   False

Circle one  T  or  F

T      F	1)  When conducting the flow test on the 240R, you can disable the audible alarm by disconnecting the middle connector from the RMS module body.

T      F	2) The CO2 scrubber can be installed with or without the center red insert as long as the package seal has not been previously opened.

T      F	3)  Under extremely cold conditions the 240R is approved to wear without the frozen ice canisters.

T      F	4) The face piece adapter must be installed with the breathing gas directional arrows facing up.

T     F	5)  The RMS must always be installed in the breathing chamber in order to assist in the proper cool of the breathing gas.

T     F	6)  Only when the red trim light is in a solid state should the user exit the area to a safe location.

T     F	7)  The constant flow rate of the 240R should be from 1.6 to 2.4 lpm

T      F	8)  The total weight of the 240R apparatus ready to use is 24 pounds

T      F	9)  The oxygen bottle should be charged to 596 liters of medical grade oxygen @ 3000psi

T     F     10)  The 9 volt battery should be changed out every 200 hours or 6 months, which every comes first.

T      F    11)   The top center section has a total of six snap slide latches

T      F    12)  The vent valve assembly has six components to it

T     F     13) When the 240R over pressurizes the vent valve will release unused oxygen.

T     F     14) The three diaphragm springs are what maintain the positive pressure on the 240R. the two upper springs have a much higher spring tension than the one lower spring.

T     F     15) The low oxygen alarm must activate between 650 –1000psig and is indicated by a flashing red light and audible alarm.

T     F     16)  The RMS will automatically power down once the system pressure

has dropped below 650psi.

T     F     17) With the exception of the center section o-ring and the vent valve

o-rings, it is recommended to inspect and re-lubricate all other o-rings

every year.

T     F      18)  There are three moisture control sponges which should be installed

Completely dry during use of the SCBA. After the ice canisters are

Installed the condensation will wet the sponges.

T     F     19)  The trim indicator just below the external gage gives the user a light

Indication of green for go, red for no go and yellow if the user has

Forgotten to put the ice in the unit before use.

T     F     20)  When training with the 240R, it is permissible to use only one CO2

Cartridge.

Part 2 	     Multiple Choice Questions

Circle the correct Letter. Circle only one letter for each question

- Which of the following will not cause high breathing resistance during

inhalation:

  - facepiece exhalation valve sticking open.

  - diaphragm springs in breathing chamber are damaged or missing.

  - demand valve in breathing chamber has failed.

  - Face piece inhalation valve is sticking closed

- Which would not be a Probable cause of the breathing gas being  uncomfortably warm during use:

  - PMC canister is missing

  - Frozen ice canisters not installed

  - User is unfamiliar with conditions of Biopak supplied atmospheres

  - Low constant oxygen flow

  - User is working in high ambient temperatures

- To conduct a low pressure leak test, which of the following steps are incorrect:

  - Install the leak test plug into the facepiece adapter and attach one

End of the test kit rubber tubing to the leak test plug and the other

End to the input port of the test kit.

  - Insert one test key in the back of the lower housing

  - Open the oxygen cylinder valve

  - Close the oxygen cylinder valve and depress the bypass valve

  - Immediately use the test kit bleed valve to adjust the SCBA

Pressure to 6-8 inches water column pressure.

  - When the test gage stabilizes wait one minute, the pressure gauge

Of the test kit shall indicate no less than 0.2 inches water column

Pressure below the stabilized starting pressure.

- Which of the following is not a correct specification for the 240R apparatus:

A.  Operational temperature range	5  to 110 degrees F

B.  Power:	one 6-volt D.C. battery required for the monitoring system

C.  Tidal Volume	over 6 liters

D.  Oxygen	440 liters @ 3000psi medical grade oxygen

- When conducting the Biomarine 240R bench testing at a Mine Rescue bench contest, which of the following test listed below is not called for:

  - constant flow test

  - demand valve test

  - emergency bypass functional test

  - vent valve functional test

  - inhalation /exhalation valve test

  - low pressure leak test

  - RMS gauge and TRIM test

  - High pressure leak test

  - Low pressure alarm test

  - Visual examination

Biopak 240R Written Test Answer Sheet 2009

1)	T  		page 12	( I )

2)	F		page 11	( H )

3)	F		page 12	( H )

4)	T		page 13	( H )

5)	F		page 13	( H )

6)	F		page 16	( H )  							7)	T		page 12	( I )

8)	F		page 25	( I )

9)	F		page 25	( I )

10)	T		page 25	( I )

11)	F		page 32	( H )

12)	T		page 34	( I )

13)	F

14)	F		page 39	( I )

15)	T		page 15	( I )

16)	F		page 15	( I )

17)	T		page 18	( I )

18)	F		page 10	( H )

19)	F		page 15	( H )

20)	F		page 11	( H )

21)	A		page 23	( I )

22)	D		page 24	( I )

23)	B		page 13	( I )

24)	B		page 21	( H )

25)	E

I =  BioPak 240 Revolution Benchman instruction manual revision I

H = BioPak 240 Revolution User instruction manual revision H