2009 Colorado Mine Rescue Contest

Idaho Springs, CO

Written Test – BG-4 Apparatus Bench

All questions are from BG4 Service Manual (Rev. 4)

Name____________________________	Company___________________________

Team Name_______________________	Contest Position No.__________________

Team Member No._________________

Directions: only one answer per question.  GOOD LUCK

- The mission must always be ended when the low-pressure warning output is at approximately 135 psi?

A. True			B. False

- The carbon cylinder has a life span of _____years?

A. 5			B. 10		C. 15		D. 20

3.   On the relief valve, the over pressure valve opens between 2-5 mbar?

A. True		B. False

4.  A leak in the dosage line can cause low dosage?

A. True		B. False

5.  Carbon monoxide contained in the exhaled air is 	is absorbed in the regeneration

cartridge?

A. True		B. False

6. When installing a relief valve, turn relief valve 45 degrees to the left?

A. True		B. False

7. High pressure O2 cylinders contain _____% O2?

A. 96.5			B. 97.5		C. 98.5		D. 99.5

8. When checking the inhalation valve for control function; the pressure gauge must indicate +10 psi?

A. True		B. False

9. When testing the CO2 absorber the test pressure should be between +7 mbar and + 10 mbar?

A. True		B. False

10. When checking components (equipment connector and breathing hoses, CO2 Absorber and Breathing air cooler) with the RZ tester, the pressure loss should not exceed 1.5 mbr/min?

A. True		B. False

11. The maximum drying temperatures for BG4 parts is _____˚C?

A. 50˚C		B. 55˚C		C. 60˚C		D. 65˚C

12. The relief valve activation occurs at _____PSI?

A. 85 			B. 86 			C. 87			D. 88

13. Dosage output is 1.5 to 1.9 PSI?

A. True		B. False

14. For units that are in constant use the valve disc on the exhalation and inhalation valves need to be replaced every _____?

A. yearly		B. 2 years		C. 3 years		D. 4 years

15. Drager Safety needs to perform a major overhaul of the pressure reducer every 5 years?

A. True		B. False

16. Constant dosage for the BG4 apparatus must be 1.5 to 1.8 L/min?

A. True		B. False

17. When checking the minimum valve the admission pressure of the minimum valve must be between _____ and _____mbar?

A. 0.5 and 2.0		B. 0.1 and 1.5		C. 1.0 and 2.5		D. 0.1 and 2.5

18. To check the relief valve on the BG4; set the pump on positive pressure, pump until relief valve opens, read admission pressure from the pressure gauge and it must be 2 to 7 mbar?

A. True		B. False

19. To check the residual warning the cylinder must be closed, observe display unit; the warning must react at approximately 700 psi?

A. True		B. False

20. The Panorama Nova mask has a peripheral vision of 95%?

A. True		B. False

21. Use of the ice in the cooling system is only required at ambient temperature above 0˚F and 32˚C?

A. True		B. False

22. During the initial freezing of the ice pack should be left to freeze completely at approximately -32 ˚C for at least 16 hours?

A. True		B. False

23. To check the exhalation valve for control function, set “pump” on negative pressure, seal inhalation breathing hose by pinching it by hand, with one strong stroke the pressure gauge must indicate -10 mbar?

A. True		B. False

24. A leaking minimum valve can cause a low dosage?

A. True		B. False

25. The volume of the breathing bag is 5.5 liters?

A. True		B. False