Louisiana 2009	BG4 Bench written test

Name_________________

Company____________________

1. The Panorama Nova mask is tested with positive pressure .		_______

A. True	B. False						pg43

2. The high pressure o-ring should be replaced once a year?		            _______

A. True	B. False						pg 44

3. The temperature for drying must not exceed 60 degrees C?		_______

A. true		B. False						pg45

4. Weight of the BG4 with 1.4 kg ice, mask and charged cylinder is ___lbs. _______

A, 31	B. 32	C. 33	D 34						pg. 48

5. A carbon cylinder holds 2 liters of compressed oxygen?			_______

A. true		B. False						pg48

6. Relief valve, on the reducer, activation is ___ psi?			_______

A. 78	B. 85	C. 87	D. 80						pg 51

7. Bypass output is 50 psi. ?							_______

A. True	B. False						pg 51

8. Minimum valve output is 80 psi?						_______

A. True	B. False						pg 51

9. The minimum valve utilizes high pressure from the reducer?		______

A. True	B. false						pg 53

10. The inhalation flange side of the center connector is long?		______

A. true		B. false						pg 55

11. Moisture trap opens at < 15 mbar?					_______

A. true		B. False						pg. 57

12. The breathing temperature at 240 minutes with a breathing rate		_______

30 l/min and ambient temperature at 75 degrees F is _____.		Pg59

A.93	B.95	C. 98	D.100

13. Operation with out ice pack is also permitted 0 degrees C.		______

A. True	B. false						pg.7

14. At a pressure lower than 1.4 mbar the negative pressure warning 	______

must react ?								pg 35

A. true 	B. False

15. Set “pump positive pressure” to check inhalation valve.			______

A. True	B. False						pg 35

16. To check the relief valve, an opened cylinder and bypass can be	_______

used to inflate the bag ?						pg 37

A. True	B. False

17. The constant dosage must be between 1.5 and 1.9 L/sec .?		_______

A. True	B. False						pg 38

18. After minimum valve check the RZ remains at position			_______

“Pump negative pressure”.						Pg 38

19 Minimum Valve test, observe pressure gauge; the admission of the	_______

Minimum valve must activate before 1.4 bar?			pg 38

20 To check the bypass valve, set the RZ on leak test?			_______

A. true		B. false						pg 39

21. Residual warning failure can be caused by a discharged battery?	_______

A. True	B. False						pg 41

22. The o-ring on the plug in connector should be replaced every 3 years.  _______

A. true 	B False						pg 44

23. The maximum temperature for washing harness is 60 degrees C.	_______

A. True	B. False						pg 45

24. Continuous operation of the BG4 at temperatures higher than 86	_______

Degrees F will result in reduced breathing air cooling.		Pg 48

A. true		B. False

25. Volume of the breathing bag is 5.5 Quarts.				_______

A. True	B. False						pg 48

Answer sheet

1.A

2.A

3.A

4.C

5.A

6.C

7.B

8.B

9.B

10.B

11.B

12.C

13.A

14.A

15.A

16.A

17.B

18.A

19.B

20.A

21.A

22.B

23.A

24.A

25.B