2009 CMR Mine Rescue Contest

Kellogg, ID

Written Test – Mine Rescue Field Competition

Name____________________________	Company___________________________

Team Name_______________________	Contest Position No.__________________

Team Member No._________________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

1.	When the barometer falls, this means:

A.   Gases will diffuse more quickly.

B.   The atmospheric pressure is rising.

C.   It is much easier for explosive gases to build up.

D.   Gases are squeezed into a smaller area.

2.	Methane is most explosive at what range?

A.   5% to 15% with at least 12.1% oxygen

B.   9.5% to 12.5% with at least 12.1% oxygen

C.   5% to 10%

D.   9.5% to 10%

3.	The safest procedure for getting survivors out of the refuge chamber or barricade is:

A.   Build an airlock

B.   Use self-rescuers

C.   Advance fresh air

D.   Break into the barricade quickly

4.	When you are judging the toxicity of a gas by its TLV, the rule of thumb is: the higher the number, the more toxic the gas is.

A.   True			B.   False

5.	An anemometer is used to measure atmospheric pressure.

A.   True			B.   False

6.	Once the coordinator at the fresh air base receives the progress report:

A.   It is recorded in a log book.

B.   The fresh air base attendant makes a decision on whether to continue.

C.   The information is then relayed on to the command center.

D.   All of the above.

7.	Tiny particles of solid and liquid matter suspended in air is called:

A.   The damps

B.   Smoke

C.   Gases

D.   All of the above

8.	"Simultaneous mapping" includes:

A.   The map man's map

B.   The fresh air base coordinator's map

C.   The command center's map

D.   All of the above

E.   Only A and B

9.	Which of the following methods is not used to help you see better when exploring in smoke?

A.   Remove your cap lamp and hold it at waist level.

B.   Use the "flood spot" if your light has one.

C.   Use a high intensity light that is used in some mines.

D.   Let the lamp hang from the cord to light up the rail or side that you're
       moving along.

10.	If nothing else is available, a smoke tube can always be used to determine air velocity.

A.   True			B.   False

11.	________ causes you to breathe faster and deeper.

A.   15% oxygen

B.   17% oxygen

C.   2% or higher carbon dioxide

D.   Both B and C

12.	Blasting produces:

A.   carbon monoxide

B.   hydrogen sulfide

C.   nitrogen dioxide

D.   all of the above

13.	Electrical malfunctions can produce oxides of nitrogen.

A.   True			B.   False

14.	In natural ventilation, air flows because of the natural pressure.  This pressure can only be supplied by thermal energy due to temperature differences.

A.   True			B.   False

15.	Usually, the measured distance using a smoke tube is ____ feet.

A.   100

B.     50

C.     75

D.     25

- 16.      The briefing committee is generally composed of:

A.   company officials

B.   federal officials

C.   state and union representatives where applicable

D.   A and B

E.   all of the above

- 17. 	Asphyxiating gases:

A.   can, in all cases, be tasted, smelled, or seen

B.   cause suffocation

C.   cause the metal parts of an apparatus to corrode

D.   do not produce an oxygen deficient atmosphere

- 18.	An elevated concentration of nitrogen in mine air can be harmful because:

A.   it can lower the oxygen content of the air

B.   it is highly explosive

C.   it is highly toxic

D.   all of the above

- 19.       Debriefings are held to:

A.   inform news reporters of developments

B.   inform family members of developments

C. review the rescue team’s findings after it has returned from
      underground

D.   all of the above

- 20.	Chemical analysis has shown hydrogen sulfide.  You should suspect:

A.   PVC or conveyor belt burning

B.   penetration into an oil or gas well

C.   detonation and burning of explosives

D.   none of the above

- 21.	In some mines the fire doors will close automatically when the ____ in the air reaches a certain level.

A.   carbon dioxide

B.   oxygen deficiency

C.   temperature

D.   carbon monoxide

- 22.	The explosive range of hydrogen is _______ to 74.2 percent.

A.   7 percent

B.   5 percent

C.   4 percent

D.   3 percent

- 23.  	Stinkdamp contains the following mixtures.

A.   hydrogen sulfide and air

B.   sulfur dioxide and air

C.   nitrogen dioxide and air

D.   none of the above

- 24.	The purpose of a(n) ___________is to separate two different atmospheres while                                                                                                 still permitting miners to enter and exit without mixing the atmospheres.

A.   regulator

B.   overcast

C.   air lock

D.   bulkhead

25.		Who decides how a fire should be fought?

A.   command center

B.   team captain

C.   fresh air base

D.   mine rescue team

- 26.	The TLV for carbon monoxide is 50 ppm or:

A.   0.5 percent

B.   0.05 percent

C.   0.005 percent

D.   none of the above

- 27.	An anemometer is a small windmill device with a mechanical counter that measures air velocities of over ______ feet per minute.

A.   100

B.   110

C.   120

D.   none of the above

- 28.	The symbol  =  on the mine map indicates.

A.   conveyor belt

B.   permanent bulkhead

C.   temporary bulkhead

D.   none of the above

- 29.	Air locking into a seal should never be undertaken until the oxygen content of the air behind the seal has been reduced to at least _____ percent.

A.   one

B.   two

C.   three

D.   none of the above

- 30.	Hazards of direct firefighting include all of the following except:

A.   explosive gases

B.   toxic gases

C.   elevated oxygen readings

D.   bad ground conditions

E.   all are hazards of direct firefighting

- 31.	The best way to relieve barricaded survivors of psychological stress is to:

A.   get them out as soon as possible

B.   communicate with them as soon as possible

C.   restrain them

D.   keep them busy

- 32.	________and_________ are highly water soluble gases and both may be released from water.

A.   oxygen and sulfur dioxide

B.   nitrogrn dioxide and hydrogen

C.   sulfer dioxide and radon

D.   hydrogen sulfide and air

- 33.	Erecting a temporary bulkhead before passing through a door or bulkhead where conditions are definitely not known will ______?

A.    provide an airlock.

B.    ensure atmospheres on each side of the structure will not mix.

C.    not alter the current status of the ventilation system.

D.    all of the above.

- 34.	Acetylene is not explosive but will burn.

A.   True			            B.   False

35.	The components of blackdamp are:

A.    hydrogen sulfide and air

B.    carbon dioxide, nitrogen, and air

C.    carbon monoxide, nitrogen, and oxygen

D.    none of the above

Answer Key for CMR Mine Rescue Field Competition - Written Test

Q                               A	IG6 - Module		Page

1.                               A	2			12

2.                               D	2			43

3.                               C	6			6

4.                               B	2			17

5.                               B	3			27

6.                               C	4			49

7.                               B	2			49

8.                               D	4			51

9.                               B	4			43

10.                             B	3			27

11.                             D	2			27, 30

12.                             D	2			32, 35, 37

13.                             A	2			35

14.                             A	3			9

15.                             D	3			31

16.                             E		4			24

17.                             B		2			19

- 18.	    	          A				2			28

- 19.		          C				4			57

- 20.                             B				2			39

- 21.	                      D				3			21

- 22.                             C				2			71

- 23.                             A				2			53

- 24.                             C				3			38

- 25.                             A				5			24

- 26.                             C				2			18

- 27.                             C				3			27

- 28.                             B				3			12

- 29.                             B				7			12

- 30.                             C				5			28 – 32

- 31.                             B				6			10

- 32.		          C				2			71

- 33.                             D				4			44

- 34.                             B				2			71

35.                             B				2			51