Colorado Mine Rescue Contest

July 21-23, 2009

Written Test

Name:________________________Team:___________________

- Mines are required to keep exposure to radiation below ___ WLM (working level months) per year.

  - 2

  - 3

  - 4

  - 5

- Rock-strata gas, commonly called rock gas is assumed to be largely nitrogen and _________________.

  - carbon monoxide

  - hydrogen

  - hydrogen sulfide

  - none of the above

- Afterdamp is toxic but not oxygen-deficient.

  - True

  - False

- Exhaust fans are used mostly in mines having little overburden.

  - True

  - False

- Natural ventilation is widely used in mines.

  - True

  - False

- The specific gravity of nitrogen dioxide is:

  - 1.0494

  - 1.5896

  - 1.5894

  - 1.5628

- There are ______ important factors that help determine where to establish a fresh air base.

  - two

  - three

  - four

  - none of the above

- A 30 pound fire extinguisher will normally last 18-35 seconds

  - True

  - False

- An example of second priority in triage would be:

  - severe bleeding

  - airway problems

  - moderate shock

  - None of the above

- The mine clerk should provide materials for additional telephone communications as needed.

  - True

  - False

- The specific gravity of butane is:

  - 2.2654

  - 2.0100

  - 2.2345

  - 2.1001

- Acetylene has a garlic taste:

  - True

  - False

- An increase in temperature and pressure will cause a gas to expand:

  - True

  - False

- The degree to which a toxic gas will affect you depends on____factors.

  - 2

  - 3

  - 4

  - none of the above

- Radon is mostly found in copper mines

  - True

  - False

- The mine supervisor should:

  - provide suitable transportation for people and supplies, as needed

  - record the equipment issued and returned

  - give each person going underground a check number

  - all of the above

- Wheeled extinguishers can weigh from 75 to 250 pounds:

  - True

  - False

- The fresh air base coordinator has _______ main responsibilities during team exploration.

  - Two

  - Three

  - Four

  - Five

- The specific gravity for carbon dioxide is:

  - 1.5292

  - 1.1591

  - 1.1529

  - 1.5291

- Because it’s highly soluble in water, test for hydrogen when stagnant water is disturbed.

  - True

  - False

- A tire fire would be considered a class_____fire.

  - A

  - B

  - C

  - D

- High expansion foam is made by mixing water and foam concentrate or detergent in a foam generator:

  - True

  - False

- Very tiny amounts of sulfur dioxide (________percent or less) will irritate your eyes and respiratory tract.

  - 0.0001

  - 0.001

  - 0.01

  - none of the above

- The time a team spends underground is usually limited to:

  - one hour or less

  - two hours or less

  - three hours or less

  - four hours or less

- Mine rescue teams should never alter ventilation without direct orders from the:

  - captain

  - chief electrician

  - command center

  - all of the above

- A high-velocity anemometer is used for measuring velocities from 1,000 to 12,000 feet per minute

  - True

  - False

- Usually, after temporary seals are erected, a waiting period of about _____ hours is recommended before beginning construction on permanent seals:

  - 24

  - 36

  - 48

  - 72

- The sound-powered communication system’s wire or cable must be at least 1,500 feet long:

  - True

  - False

- Air locking operations should always begin on the intake side of the fire.

  - True

  - False

- Recovery by direct ventilation is very similar to advancing a fresh air base:

  - True

  - False

- Spatial disorientation can occur when you travel in smoke

  - True

  - False

- Located at the top of the chain-of-command is the:

  - MSHA supervisor

  - mine manager

  - chief engineer

  - none of the above

- Portable communication systems used for mine rescue work must be approved under 30 CFR, Part 27.

  - True

  - False

- Prior to a mine rescue team passing through a door or bulkhead behind which conditions are not definitely known they should:

  - open a compressed bottle of oxygen

  - erect an air lock to prevent mixing of atmospheres

  - open the door slowly

  - none of the above

- Survivors who are showing signs of hysteria should be left alone until they calm down.

  - True

  - False