2009 Northern Mine Rescue Contest

Rochester, NY

Written Test – Mine Rescue Field Competition

Name____________________________	Company___________________________

Team Name_______________________	Contest Position No._________________

Team Member No._________________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

- While rescue teams are working underground, the main fan (or fans) should be monitored or guarded by an authorized individual.

A.  True
B.  False

- In order for flammable gas to explode, there must be: 

A.  Enough of the gas in the air
B.  Enough oxygen
C.  A source of ignition
D.  A and C only
E.  A, B, and C

- ____________________ is used to channel intake air into a working area that otherwise wouldn’t have adequate ventilation.

A.  Check Curtain
B.  Line Brattice
C.  Bulkhead
D.  None of the above

- If the barometer rises, indicating increased pressure, gas responds by contracting.

A.  True
B.  False

- When exploring a mine, your first priority is the rescue of survivors.

A.  True
B.  False

- The specific gravity of normal air is:

A.  2.0
B.  2.5
C.  1.0
D.  3.7

- You shouldn’t drink alcoholic beverages for at least _________ hours before you get under oxygen.

A.  5 to 6
B.  7 to 8
C.  9 to 10
D.  12 to 18

- Class C fires involve flammable or combustible liquids such as gasoline, diesel fuel, kerosene, and grease.

A.  True
B.  False

- The degree to which a toxic gas will affect you depends on:

A.  How concentrated the gas is.
B.  How toxic (poisonous) the gas is.
C.  How long you’re exposed to the gas.
D.  A and C only
E.  A, B, and C

- The sorting of victims is commonly referred to as a “triage” system.
A.  True
B.  False

- If the team is stopped at one location for an extended period, or if it is performing a specific task, it’s common practice for the fresh air bas to signal one long pull about every _______ minutes to check on the team’s condition.

A.  3
B.  5
C.  10
D.  15
E.  None of the above.

- The most hazardous aspect of hydrogen is that in high concentrations it can replace oxygen in the air and act as an asphyxiant.

A.  True
B.  False

- ___________________ is produced by all fires because of the incomplete combustion of carbon materials during the burning process.

A.  Carbon Monoxide
B.  Carbon Dioxide
C.  Hydrogen Sulfide
D.  All of the above
E.  None of the above

- Re-establishing ventilation and bringing fresh air to an area damaged by fire or explosion is the main task of mine rescue teams in a recovery operation.

A.  True
B.  False

- A _______________ percent concentration of oxygen will cause dizziness and headaches.

A.  5
B.  10
C.  15
D.  21
E.  None of the above

- ______________ will neither burn nor explode.

A.  Nitrogen
B.  Carbon Monoxide
C.  Carbon Dioxide
D.  A and C only
E.  A, B, and C

- Maintaining communications with the working rescue team and the command center is one of the main responsibilities of the fresh air base coordinator.

A.  True
B.  False

- The symbol (	         ) indicates ____________ on a ventilation map.

A.  Direction of exhaust air
B.  Line Brattice
C.  Direction of intake air
D.  Temporary bulkhead
E.  A or C

- The tightness of the seals is one of the main factors governing the time for unsealing a fire area.

A.  True
B.  False

- The explosive range of propane in normal air is:

A.  3 to 12.5 percent
B.  1.86 to 8.41 percent
C.  12.3 to 18.4 percent
D.  2.12 to 9.35 percent

- Examples of second priority conditions in the triage system are:

A.  Minor bleeding
B.  Moderate Shock
C.  Fractured arm, hand, or foot
D.  Abrasions
E.  All of the above

- A jump in the pressure recording chart for the main fan may be an indication of an explosion.

A.  True
B.  False

- A regulator is a device used to control the direction of airflow.

A.  True
B.  False

- _____________ is a mixture of carbon dioxide, nitrogen, and air.

A.  Whitedamp
B.  Afterdamp
C.  Blackdamp
D.  Firedamp
E.  None of the above

- The captain or co-captain may mark the team’s route with chalk or spray paint at key points.

A.  True
B.  False

- The more muscular the individual, the longer it will be before rigor mortis will develop.

A.  True
B.  False

- __________ has a slight garlic odor.

A.  Nitrogen dioxide
B.  Sulfur dioxide
C.  Propane
D.  Acetylene
E.  None of the above

- During direct firefighting _____________ are hazards to the team:

A.  Electrocution
B.  Toxic Gases
C.  Oxygen Deficiency
D.  All of the Above
E.  None of the Above

- When a team locates a body, you only need to mark the location on the mine map.

A.  True
B.  False

- A monoammonium phosphate extinguisher is effective in fighting:

A.  Class A fires
B.  Class B fires
C.  Class C fires
D.  All of the above

- Debriefings are held to:

A.  Inform news reporters of developments
B.  Inform family members of developments
C.  Review the rescue team’s findings after they have returned to the fresh air base.
D.  All of the above
E.  None of the above

- The most likely source of ethane, propane, or butane in a mine is: 

A.  Use of diesel equipment
B.  Battery charging stations
C.  Leakage from adjacent gas or oil wells
D.  All of the above
E.  None of the above

- Stinkdamp is the mixture of hydrogen sulfide and air.

A.  True
B.  False

- The team captain can alter ventilation if he feels it is necessary.

A.  True
B.  False

- If fire begins to back up against the flow of intake air in search of oxygen, a transverse brattice can be installed from side to side in the passageway, with open space near the back.

A.  True
B.  False