Kansas Shoot-Out

May 12-14, 2009

Written Test

Name:__________________________Team:____________________

- As little as ______% carbon monoxide can cause death in three hours.

  - 0.005

  - 5.000

  - .01

  - .05

- Hydrogen sulfide is found in high places in the mine because it is a relatively light gas?

  - True

  - False

- The explosive range for carbon monoxide, in normal air, is 12.5% to 74.5%

  - True

  - False

- A medium-velocity (or “regular”) anemometer is used for measuring velocities from 20 to 2,000 feet per minute.

  - True

  - False

- ______________is the reventilation of the entire sealed area at once.

  - building bulkheads

  - direct ventilation

  - progressive ventilation

  - not of the above

- Methane is most explosive in the 9.5% to 11% range

  - True

  - False

- __________________ of Sufur Dioxide is dangerous to life

  - 0.04% to 0.05%

  - 1 ppm to 10 ppm

  - 2% to 5.5%

  - 4 ppm to 6 ppm

- The components of blackdamp are:

  - carbon monoxide, nitrogen, and air

  - hydrogen sulfide, methane, and air

  - carbon dioxide, methane, and air

  - not of the above

- Gases that are neither toxic nor explosive:

  - can be dangerous because they can displace oxygen

  - are not detectable with detection equipment

  - are not dangerous

  - are not found in mine atmospheres

- Dry chemical fire extinguishers put out fires by stopping the _______________between the fuel and the oxygen

  - heat reaction

  - flame reaction

  - chemical reaction

  - none of the above

- When operating a hand-held extinguisher, you should stay high and try to get within 6 to 8 feet of the fire before turning on the fire extinguisher

  - True

  - False

- Gas Readings should be taken:

  - at all intersections

  - on the other side of bulkheads or doors prior to traveling through

  - at the furthest point of travel in any passageway

  - all of the above

- When directly fighting a fire the team should always approach the fire from the intake side:

  - True

  - False

- Air locking into a seal should never be undertaken until the oxygen content of the air behind the seal has been reduced to at least ______percent

  - 2

  - 2.5

  - 3

  - 5.5

- A 30-pund fire extinguisher will normally last 15 to 25 seconds

  - True

  - False

- Of the following gases which one is not soluble in water:

  - sulfur dioxide

  - methane

  - hydrogen

  - none of the above

- One of the duties of the chief engineer is to prepare an inventory of existing equipment and supplies:

  - True

  - False

- In accordance with the freedom of information act, news reporters should not be restricted from wandering about the mine property:

  - True

  - False

- The specific gravity for carbon monoxide is:

  - 0.9671

  - 0.9651

  - 0.9567

  - 0.9672

- Radon has:

  - a specific gravity of 7.516

  - is not explosive

  - is not soluble in water

  - all of the above

- The human body is what percent fluid:

  - 75

  - 80

  - 85

  - 90

- The speed or rate at which you will travel underground is usually determined by the:

  - command center

  - fresh air base

  - team captain

  - size of the mine

- _______________ provides the fresh air base and the command center with a visual record of what is happening underground:

  - a MINER approved linkline

  - briefing session

  - simultaneous mapping

  - the mine phone

- A positive indicator that a fire exists in a mine is:

  - methane and carbon dioxide in the exhaust airway

  - carbon monoxide and/or smoke in the exhaust air way

  - lowered oxygen content in the exhaust

  - a disruption in normal ventilation

- Before going into a mine to explore for, or to fight a fire, the team should:

  - ensure gas tests are being done at the exhaust

  - make sure the fan is guarded and monitored

  - make sure the main fan is running

  - all of the above

- High expansion foam is only useful in fighting class___________fires:

  - A and B

  - A and C

  - B and C

  - none of the above

- Roof falls are not likely in areas that have been inundated with water:

  - True

  - False

- A mine rescue and recovery simply consists of a team going into a mine to rescue survivors or to put out a fire:

  - True

  - False

- Basically, there are two ways to erect brattice seals:

  - True

  - False

- Hazards that a rescue team should consider when fighting a mine fire directly include:

  - oxygen deficiency

  - water inundation

  - rib failure

  - all of the above

- Portable communication systems used for mine rescue must be approved under 30 CFR, Part:

  - 18

  - 20

  - 23

  - 27

- Equipment for exploration work falls into two categories:

  - True

  - False

- The best way to relieve psychological stress in survivors is ignore them.

  - True

  - False

- A temporary seal cannot be built using wood.

  - True

  - False

- It is recommended that you drink alcoholic beverages for at least ________ hours before you go under oxygen.

  - 8 to 10

  - 10 to 16

  - 12 to 18

  - none of the above