Rolla MO

Written test

2009

- When making an air velocity measurement with an anemometer, you should hold the instrument at arm’s length in front of you with the air entering the dial side.

  - True

  - False

- Under normal conditions the only two gases that will be found at a mine is Oxygen and Nitrogen.

  - True

  - False

- The rate of gas diffusion is decreased by higher velocities of air currents.

  - True

  - False

- A decrease in pressure causes a gas to expand.

  - True

  - False

- High temperatures (or heat) cause gases to expand, so they diffuse more slowly.

  - True

  - False

- Carbon Monoxide (CO) is explosive and flammable. Its explosive range in normal air is

  - 5.5 to 10.5 percent

  - 12.5 to 50 percent

  - 12.5 to 74.2 percent

  - 15.5 to 75 percent

- Carbon Monoxide (CO) is explosive and flammable but not toxic.

  - True

  - False

- Which of the following gas is not a heavy hydrocarbon

  - Ethane

  - Propane

  - Butane

  - Acetylene

- Sulfur Dioxide gas in concentrations as low as ______ is dangerous to life.

  - .02 to .07 %

  - .01 to .05%

  - .04 to .05%

  - .4 to .5%

  - .004 to .005%

- The chemical symbol for acetylene is

  - C2H4

  - C3H6

  - C2H2

  - C4H4

  - A2C4

- If you took a ventilation reading in a drift 14 feet wide and 16 feet high and record a true reading of 1200 feet per minute, what are the cubic feet per minute?

  - 2400 cfm

  - 250,000 cfm

  - 268,800 cfm

  - 286,800 cfm

- Taking a reading in a drift 10 feet wide and 10 feet high and a true reading of 1000 feet per minute the cubic feet per minute would be.

  - 1000 cfm

  - 10,000 cfm

  - 100,000 cfm

  - None of the above

- Requirements for a fresh air base are

  - Positive ventilation, travel way for people and supplies, communication reel

  - Positive ventilation, Travel way for people and supplies, communication with command center, communication with team

  - Fresh air, Communication with command center, communication with team

  - None of the above

- When a team goes into a mine to explore or fight a fire, it should be concerned with two main things.

  - Is there a back up team and are they trained

  - Spreading of the fire and is there a second way out

  - Spreading of the fire and the possibility of an explosion

  - Explosions and poor ventilation

  - Is the team properly trained for this task and do they have all the equipment needed.

- Before going underground, the team should make sure that the main fan is running, that a guard is monitoring the operation of the fan and

  - The command center knows where they are going

  - A back up team is able and ready to enter the mine

  - Tests are being made at the main exhausts for any gases that may be present

  - The command center will not let anyone else in the mine until your team returns to the FAB.

- If a survivor is found in contaminated or questionable air and is fully conscious and ready to travel , the team should

  - Get them to fresh air as soon as possible

  - Let them continue to head for fresh air so the team can do more important work

  - Provide the person with breathing protection and send them toward fresh air.

  - None of the above

- If the team finds more then one survivor in a refuge chamber the team should.

  - Do a triage and decide who should leave first, provide breathing protection and send the person to fresh air via shortest course.

  - Do a triage and decide who should go first, leave a team member with the remaining person to keep him calm and ensure him the team will be returning.

  - Do a triage and load the first selected person onto a stretcher, provide breathing protection and transport to the nearest fresh air base, and repeat process until all survivors are rescued.

  - Provide w-65 self rescuers to all of the survivors and get them to fresh air as soon and as quick as they can.

- In body recovery, a muscular individual will begin putrefaction sooner then an obese person.

  - True

  - False

- If a body is found the first thing a team member should do is check all of his/her pockets and look for stuff that will identify him/her.

  - True

  - False

- Characteristics of Methane (CH4) are

  - A little bit lighter then air

  - 0.55 specific gravity

  - A little bit heavier then Hydrogen (H2)

  - A little bit lighter then  Carbon Monoxide (CO)

  - All of the above

- The fact that Nitrogen Dioxide (NO2) is non toxic when encountered by a team they should map and mark it but move on and worry about more dangerous gases.

  - True

  - False

- Hydrogen Sulfide (H2S) is extremely poisonous, at .005 to .010% causes inflammation of the eyes and respiratory tract. .02 to .07% can lead to bronchitis or pneumonia. .07 to 10% can cause rapid unconsciousness, cessation of respiration, and death. .10 to .20% or more can cause rapid death.  Additionally it can be absorbed through the skin and be fatal upon contact.

  - True

  - False

- The specific gravity of (H2) is 0.0695 it will be detected in _____areas of a drift.

  - High

  - Low

- H2 has an explosive range of 4.0 to 74.2% with as little as _______% oxygen.

  - 2

  - 3

  - 4

  - 5

  - None of the above

- Carbon Monoxide (CO) has an explosive range of 12.5 to 74.2 percent in_______% oxygen.

  - 5

  - 10

  - 15

  - 21

- When measuring ventilation with a smoke tube one must convert FPS to FPM. Therefore 23 seconds

60 seconds    = ____________

a. .28 minute

b. .38 minute

c. .48 minute

d. none of the above

- A medium –velocity anemometer is used to determine air velocities from ________to ________feet per minute.

  - 100/1000

  - 120/1200

  - 120/2000

  - 1000/3000

- Non-sparking tools and nails must be used in all mines to reduce the chance of a spark that could cause an ignition.

  - True

  - False

- Mine rescue apparatus and equipment shall be maintained in a manner that ensures readiness for immediate use. All apparatus must be inspected by a trained person.

  - Once a week

  - Twice a month

  - Once a month

  - Once every 90 days

  - Only before they are going to be worn

- Team signals of one signal=stop, two signals= advance, three signals=retreat, four signals= distress are international signals all team members know and use.

  - True

  - False

- The fact that an underground fire will seek and demand oxygen it may cause a problem for a team approaching the fire from up wind. To help control this ventilation issue the team can erect a

  - Transverse brattice

  - Hurdle brattice

  - Line brattice

  - Regulator

  - A and or B

- The extremely toxic gas, Carbon Monoxide, is produced my all fire because of the complete combustion of carbon materials during the burning process.

  - True

  - False

- The best way to prevent hydrogen pops during a fire fighting event is to

  - Build a fire regulator

  - Decrease air flow

  - Provide adequate air over the fire

  - None of the above

- The command center should stop or reverse the main mine fan if they believe the team is in danger

  - True

  - False

- If you suspect rubber, neoprene and PVC is burning in a mine one gas that may be present is Arsine and its maximum allowable part per million would be

  - 0.5

  - 0.05

  - 0.005

  - 5.0