COLORADO MINE RESCUE CONTEST

TMX 412 MULTI- GAS INSTRUMENT TEST

ANSWER SHEET QUESTIONS

25 QUESTIONS

JULY 20-24, 2009

FROM TRAINING MODULE IG6 REVISED 2008: MINE GASES

True and False Questions (1-10)

- A decrease in pressure causes a gas to contract

  - True

  - False (page 2-5)

- The specific gravity of sulfur dioxide is 2.2683.

  - True

  - False (page 2-6)

- The explosive range for hydrogen is 4.0 to 72.4 percent in the presence of normal air.

  - True

  - False (page 2-7)

- The Threshold Limit Value (TLV) for carbon dioxide (CO2) is 0.5 percent.

  - True (page 2-9)

  - False

- The specific gravity of  Oxygen (O2)  is 1.1054

  - True (page 2-13)

  - False

- Carbon dioxide (CO2) will burn and explode.

  - True

  - False (page 2-15, under CO2, statement says carbon dioxide will neither burn nor explode).

- The specific gravity of carbon dioxide (CO2) is 1.5291.

  - True (page 2-15)

  - False

- Carbon dioxide (CO2) is not soluble in water.

  - True

  - False (page 2-15)

- The specific gravity of nitrogen dioxide (NO2) is 1.5849.

  - True

  - False  (page 2-17)

- Hydrogen sulfide (H2S) is flammable and explosive in concentrations from 4.3 to 45.5 percent in normal air.

  - True (page 2-19)

  - False

FROM TRAINING MODULE MSHA 3027 REVISED 2008:

MINE VENTILATION Multiple Choice Questions (11-18)

- A medium-velocity or “regular” anemometer is used for measuring velocities from ___________.

  - 100 to 2000 feet per minute

  - 200 to 2200 feet per minute

  - 120 to 2000 feet per minute  (page 3-16)

  - 100 to 2200 feet per minute

- A high-velocity anemometer is used for measuring velocities from ___________.

  - 1,500 to 8,000 feet per minute

  - 2,000 to 12,000 feet per minute

  - 1,500 to 10,000 feet per minute

  - 2,000 to 10,000 feet per minute (page 3-16)

- The anemometer actually measures linear feet of travel and requires timing- usually one minute- to determine __________ in feet per minute.

  - quantity

  - quality

  - area

  - velocity (page 3-16)

- Two instruments commonly used to measure air movement are the anemometer and the _________.

  - pilot tube

  - magic wand

  - pyrometer

  - smoke tube (page 3-16)

- To obtain the quantity of the air current in cubic feet per minute, the area is then multiplied by the ______.

  - width

  - length

  - volume

  - velocity  (page 3-16)

- If your team finds a fallen check curtain while exploring a mine you should____________.

  - immediately re-hang it and make it air-tight

  - build an air lock in front of it

  - leave it as it is for the time being and report the condition to the command center  (page 3-10 first paragraph).

  - neatly fold it and lay it against a rib out of the way

- The following smoke tube reading of 10 foot measured distance and an average of 15 seconds with an area of 200 ft2 the quantity of airflow

is ____________.

  - 8,400 ft/min

  - 8,800 ft/min

  - 8,000  ft/min

  - none of the above ( (8,000 ft3/min), page 3-20)

- A commonly used method of measuring the velocity in an airway is to traverse the airway to get the ___________ velocity in the airway.

a.  lowest

b.  highest

c.   median

- average  (page 3-17, 3rd paragraph)

From MNM Contest National Mine Rescue Contest Rules 2008

- During the gas bench testing the contestant will be expected to report all 4- gas concentrations in the following order:

  - O2, CO, NO2, CH4,

  - O2, CO, CH4 NO2,

  - O2, CH4, NO2, CO

  - O2, CH4, CO, NO2  (page 46)

From Multi Gas Monitor TMX 412 Instruction Manual

- When the instrument TMX 412 is in the normal viewing mode, an _______-

segment battery status indicator continuously displays the battery condition.

  - 6

  - 7

  - 8   (page 8, last paragraph)

  - 10

- To manually activate the backlight when needed, press and release the (E) key.  The backlight will illuminate the display for approximately _____________.

  - 5 seconds

  - 15 seconds  (page 9, under 4.3, last sentence)

  - 30 seconds

  - 45 seconds

- When a monitored gas concentration reaches the low level alarm setpoint, the instrument emits a short beep approximately every ________ seconds.

  - 12

  - 1.2 (page 11, 4.5.1 low alarm)

  - 2.4

  - None of the above

- From the normal READING mode, to start the instrument zeroing process press the _____________.

  - (MODE) key

  - (MODE) key then the (E) key   (page 13, 6th and 7th paragraph)

  - (MODE) and (E) keys simultaneously

  - none of the above

- When you replace the lithium battery cells, replace with three _________ 3.0 volt lithium battery cells.

  - Duracell  DA 123A

  - Panasonic CA 123A

  - Sanyo CR123A (page 21, 1st paragraph)

  - None of the above

- When a new Oxygen sensor is installed, allow ________ for it to stabilize before attempting calibration.

  - 5 minutes

  - 15 minutes  (Answer on page 22, NOTE)

  - 30 minutes

  - 45 minutes