2009 UMR/MISSOURI REGIONAL MINE RESCUE CONTEST

ROLLA, MISSOURI

412 MULTI- GAS INSTRUMENT TEST

ANSWER SHEET (25 QUESTIONS)

MSHA 3027 (Formerly IG 6) Revised 2008

FROM TRAINING MODULE IG6 REVISED 2008: MINE GASES

True and False Questions (1-10)

- An increase in temperature causes a gas to contract

  - True

  - False (Page 2-5)

- The specific gravity of sulfur dioxide is 2.2683.

  - True

  - False (Page 2-6   s.g. is 2.2638)

- The explosive range for hydrogen is 4.0 to 72.4 percent.

  - True

  - False (Page 2-7)

- The Threshold Limit Value (TLV) for carbon dioxide (CO2) is 0.5 percent.

  - True (Page 2-9)

  - False

- Oxygen is not an explosive gas and it doesn’t support combustion.

  - True

  - False (Page 2-13)

- The specific gravity of carbon dioxide (CO2) is 1.5219.

  - True

  - False (Page 2-15)

- The specific gravity of carbon monoxide (CO) 0.9627.

  - True

  - False (Page 2-16)

- Carbon monoxide (CO) is explosive and non flammable.

  - True

  - False (Page 2-16)

- The specific gravity of nitrogen dioxide (NO2) is 1.5894.

  - True (Page 2-17)

  - False

- Hydrogen sulfide (H2S) is flammable and explosive in concentrations from 3.4 to 46.5 percent in normal air.

  - True

  - False (Page 2-19)

FROM TRAINING MODULE MSHA 3027 REVISED 2008:

MINE VENTILATION Multiple Choice Questions (11-18)

- A medium-velocity or “regular” anemometer is used for measuring velocities from ___________.

  - 100 to 2000 feet per minute

  - 200 to 2200 feet per minute

  - 120 to 2000 feet per minute  (Page 3-16)

  - 100 to 2200 feet per minute

- A high-velocity anemometer is used for measuring velocities from ___________.

  - 1,500 to 8,000 feet per minute

  - 2,000 to 12,000 feet per minute

  - 1,500 to 10,000 feet per minute

  - 2,000 to 10,000 feet per minute (Page 3-16)

- The anemometer actually measures linear feet of travel and requires timing- usually one minute- to determine __________ in feet per minute.

  - quantity

  - quality

  - area

  - velocity (Page 3-16)

- Two instruments commonly used to measure air movement are the anemometer and the _________.

  - pilot tube

  - magic wand

  - pyrometer

  - smoke tube (Page 3-16)

- To obtain the quantity of the air current in cubic feet per minute, the area is then multiplied by the ______.

  - width

  - length

  - volume

  - velocity  (Page 3-16)

- If your team finds a fallen check curtain while exploring a mine you should____________.

  - immediately re-hang it and make it air-tight

  - build an air lock in front of it

  - leave it as it is for the time being and report the condition to the command center  (Page 3-10 first paragraph).

  - neatly fold it and lay it against a rib out of the way

- The following smoke tube reading of 10 foot measured distance and an average of 15 seconds with an area of 200 ft2 the quantity of airflow is ____________.

  - 8,400 ft/min

  - 8,800 ft/min

  - 8,000  ft/min

  - none of the above  (Page 3-20, 8,000 ft3/min)

- A commonly used method of measuring the velocity in an airway is to traverse the airway to get the ___________ velocity in the airway.

a.  lowest

b.  highest

c.   median

- average  (Page 3-17, 3rd paragraph)

From Multi Gas Monitor TMX 412 Instruction Manual (19-25)

- When the instrument TMX 412 is in the normal viewing mode, an _______ segment battery status indicator continuously displays the battery condition.

  - 6

  - 7

  - 8   (Page 8, last paragraph)

  - 10

- To manually activate the backlight when needed, press and release the (E) key.  The backlight will illuminate the display for approximately _____________.

  - 5 seconds

  - 15 seconds  (Page 9, under 4.3, last sentence)

  - 30 seconds

  - 45 seconds

- When a monitored gas concentration reaches the low level alarm setpoint, the instrument emits a short beep approximately every ________ seconds.

  - 12

  - 1.2 (Page 11, 4.5.1 low alarm)

  - 2.4

  - None of the above

- The dimensions on the TMX-412 are ___________.

  - 4.5” L, 2.5”W, 2.5” H

  - 4.5” L, 2.5”W, 2.0” H

  - 4.5” L, 2.5”W, 1.75” H

  - none of the above (Page 27)

- When you replace the lithium battery cells, replace with three _________ 3.0 volt lithium battery cells.

  - Duracell  DA 123A

  - Panasonic CA 123A

  - Sanyo CR 123A (Page 21, 1st paragraph)

  - None of the above

- When a new Oxygen sensor is installed, allow _________ for it to stabilize before attempting calibration.

  - 5 minutes

  - 15 minutes  (Answer on page 22, NOTE)

  - 30 minutes

  - 45 minutes

- The temperature range for continuous operation of the TMX 412 is __________.

  - -40 degrees F to 100 degrees F

  - -20 degrees F to 100 degrees F

  - -4 degrees F to 122 degrees F (Answer on page 27)

  - None of the above