2009 UMR/MISSOURI REGIONAL MINE RESCUE CONTEST

ROLLA, MISSOURI

iTX MULTI - GAS INSTRUMENT TEST

MSHA 3027 (Formerly IG 6) Revised 2008

(25 QUESTIONS)

FROM TRAINING MODULE IG6 REVISED 2008: MINE GASES

True and False Questions (1-10)

- An increase in temperature causes a gas to contract

  - True

  - False

- The specific gravity of sulfur dioxide is 2.2683.

  - True

  - False

- The explosive range for hydrogen is 4.0 to 72.4 percent.

  - True

  - False

- The Threshold Limit Value (TLV) for carbon dioxide (CO2) is 0.5 percent.

  - True

  - False

- Oxygen is not an explosive gas and it doesn’t support combustion.

  - True

  - False

- The specific gravity of carbon dioxide (CO2) is 1.5219.

  - True

  - False

- The specific gravity of carbon monoxide (CO) is 0.9627

  - True

  - False

- Carbon monoxide (CO) is explosive and non flammable.

  - True

  - False

- The specific gravity of nitrogen dioxide (NO2) is 1.5894.

  - True

  - False

- Hydrogen sulfide (H2S) is flammable and explosive in concentrations from 3.4 to 46.5 percent in normal air.

  - True

  - False

FROM TRAINING MODULE MSHA 3027 REVISED 2008:

MINE VENTILATION Multiple Choice Questions (11-18)

- A medium-velocity or “regular” anemometer is used for measuring velocities from ___________.

  - 100 to 2000 feet per minute

  - 200 to 2200 feet per minute

  - 120 to 2000 feet per minute

  - 100 to 2200 feet per minute

- A high-velocity anemometer is used for measuring velocities from ___________.

  - 1,500 to 8,000 feet per minute

  - 2,000 to 12,000 feet per minute

  - 1,500 to 10,000 feet per minute

  - 2,000 to 10,000 feet per minute

- The anemometer actually measures linear feet of travel and requires timing- usually one minute- to determine __________ in feet per minute.

  - quantity

  - quality

  - area

  - velocity

- Two instruments commonly used to measure air movement are the anemometer and the _________.

  - pilot tube

  - magic wand

  - pyrometer

  - smoke tube

- To obtain the quantity of the air current in cubic feet per minute, the area is then multiplied by the ______.

  - width

  - length

  - volume

  - velocity

- If your team finds a fallen check curtain while exploring a mine you should____________.

  - immediately re-hang it and make it air-tight

  - build an air lock in front of it

  - leave it as it is for the time being and report the condition to the command center

  - neatly fold it and lay it against a rib out of the way

- The following smoke tube reading of 10 foot measured distance and an average of 15 seconds with an area of 200 ft2 the quantity of airflow is ____________.

  - 8,400 ft/min

  - 8,800 ft/min

  - 8,000  ft/min

  - none of the above

- A commonly used method of measuring the velocity in an airway is to traverse the airway to get the ___________ velocity in the airway.

a.   lowest

b.   highest

c.   median

- average

From Multi Gas Monitor iTX Instruction Manual (19-25)

- In the gas reading mode of the iTX, as the instrument battery life is reduced, the shaded area of the battery indicator will turn clear until the instrument reaches the low battery condition.  With _______ minutes of battery life left, a low battery condition occurs.

  - 5

  - 10

  - 20

  - None of the above

- iTX units equipped with bias sensors will self discharge a fully charged battery in approximately in four days.  Because of this, bias sensor equipped units are shipped without the battery installed.  Upon receipt, install the battery and allow the bias sensors to stabilize for _______ hours.

  - 12-24

  - 16-32

  - 24-52

  - None of the above

- If the PPM Explosive gas feature is enabled, this screen will display the current concentration of gas in parts per million (ppm) in ______.

  - 1 ppm intervals up to 999 ppm

  - 2 ppm intervals up to 999 ppm

  - 5 ppm intervals up to 1,000 ppm

  - 50 ppm intervals up to 10,000 ppm

- The dimensions on the iTX are ___________.

  - 4.5” L, 3.0”W, 2.0” H

  - 4.75” L, 3.19”W, 1.68” H

  - 4.5” L, 2.5”W, 1.75” H

  - none of the above

- Remove the battery pack and replace with _____ fresh alkaline batterie(s).  A replacement cell alkaline battery pack is available  for use with the iTX.

  - 1 AAA

  - 2 AAA

  - 3 AAA

  - None of the above

- When a new NO bias sensor is installed, you must turn on the instrument so the iTX identifies that this bias sensor is installed and recognizes the need to supply a bias voltage.  Then turn off the unit and put it on charge for _______ hours to allow the sensor to stabilize before calibration.

  - 6 to 12

  - 12 to 24

  - 24 to 48

  - None of the above

- The operating temperature range for typical toxic/oxygen (not LEL) sensors of the iTX is __________.

  - -40 degrees F to 100 degrees F

  - -20 degrees F to 100 degrees F

  - -4 degrees F to 122 degrees F

  - None of the above