Team Trainer Test

New Iberia LA. 2009

- The Mine Manager or Superintendent usually carries full responsibility for the rescue and recovery operation.

A. true	b. false

- MSHA will always be totally in charge of all mine rescue and recovery work.

  - True	b. False

- A mine rescue team member will be ineligible to serve on a team if ________hours are missed during a one year period.

  - 4 hours

  - 6 hours

  - 8 hours

  - 10 hours

- A mine rescue team member must receive underground training once each ________ months.

  - 2 months

  - 4 months

  - 6 months

  - Once a year.

- A mine rescue team member must wear a breathing apparatus and be under oxygen for two hours every _________month(s).

  - Month

  - Two

  - Three

  - Four

- A MSHA District Manager can revoke a mine rescue instructor’s approval to instruct for good cause. Once decided by the district manager this is a final decision.

  - True	b. False

- During an emergency MSHA will issue a 103A order. This order is intended to protect miners and prevents the company from doing anything without MSHA approval.

  - True		b. False

- An elevated concentration of carbon dioxide (CO2) in a mine can be harmful because:

  - it is highly explosive

  - it increases the breathing rate

  - it is highly toxic in small concentrations

  - all of the above

- Oxides of nitrogen can occur in a mine atmosphere:

  - When certain explosives are used

  - When diesel-powered equipment is being used

  - When electric equipment produces arcs or sparks

  - All of the above

- Characteristics of hydrogen sulfide include:

  - Explosive

  - Highly toxic

  - Can be liberated from pools of stagnant water

  - All of the above

- Atmospheric pressure and temperature are important factors because they:

  - Affect the rate of diffusion of a gas by ventilation

  - Can cause false readings on gas detection instruments

  - Lower oxygen content in the mine

  - All of the above

- Two ways to best minimize the stress associated with responding to emergencies are to work closely with other more experienced responders and:

  - Stay out of harms way

  - Medication

  - Practice your skills often

  - Don’t recognize stress as being a problem

- Infectious diseases range from such generally mild conditions as the common cold to life-threatening diseases such as tuberculosis. The four diseases of most concern to Emergency medical Responders are:

  - Human immunodeficiency virus, Hepatitis, Tuberculosis, Meningitis

  - AIDS, Hepatitis, Blood borne pathogen,  Influenza

  - Typhoid fever, Viral encephalitis,  Bacterial Meningitis, SARS

  - Hepatitis, Tuberculosis, Meningitis,  Lyme disease

- The appropriate rate if compressions during adult CPR is_____per minute.

  - 80 to 100

  - No faster then 80

  - About 100

  - No faster 120

- Before providing rescue breaths, you must check for the presence of adequate breathing. Do this by listening for air flow from the patients nose mouth and by:

  - Shaking the patient

  - Looking for chest rise

  - Observing pupil response

  - Sweeping the mouth for obstructions

- Pulmonary resuscitation is:

  - Any effort to restart normal heart rhythms

  - Any effort to revive or restore normal breathing

  - The use of mechanical devices to restart breathing

  - The ability to restore normal heart rhythm and breathing

- Mine rescue teams should alter existing ventilation:

  - Only when directed to do so by the command center.

  - When the team captain decides to do so.

  - When they encounter high concentrations of carbon monoxide

  - When they encounter smoke.

- Temporary bulkheads built in a passageway should be placed at least 4 to 6 feet into the passageway in order that;

  - Sufficient space is available to construct a permanent bulkhead.

  - It will be protected from further explosions

  - It will not be affected by fire if a fire should spread to that passageway.

  - All of the above

- Barefaced exploration should be attempted only when:

  - No breathing apparatus is available

  - Miners are trapped in the mine

  - A backup mine rescue team with apparatus is immediately available

  - A fresh air base is established.

- In sealing a fire it is recommended that:

  - Only permanent bulkheads be used

  - The last intake and last exhaust be sealed simultaneously

  - A single air sample tube be used

  - All of the above

- Burning materials that give off extremely toxic gases in addition to carbon monoxide

  - Timbers

  - Hydraulic fluids

  - Neoprene and other synthetic rubber compounds

  - All of the above

- The preferred type of extinguisher for teams is a dry chemical type that contains:

  - Sodium bicarbonate

  - Potassium chloride

  - Carbon tetrachloride

  - Monoammonium phosphate

- Copper tubes or pipes are inserted in temporary and permanent bulkheads for the purpose of:

  - Checking for smoke

  - Bleeding off excess pressure from the sealed area

  - Collecting air samples from the sealed area

  - Ventilating the sealed area

- After an explosion has occurred, the hazards a team might encounter are:

  - Spreading fires

  - Further explosions

  - Weakened ground conditions

  - All of the above

- In the Code of Federal regulations 49.1 “Availability of mine rescue teams” clearly outlines the requirements of a mine rescue team except teams operating under 49.3 or 49.4.

  - True

  - False