Ruidoso Mine Rescue Contest

New Iberia

April 30, 2008

Team Trainer Test

___

1.	If you know the specific gravity of a gas, you will know where it will be located in the mine and where you should test for it.

a.  True

b.  False

- It is much easier to remove a concentration of sulfur dioxide by ventilation than it is to remove the same concentration of hydrogen.

a.  True

b.  False

- During recovery of previously sealed areas, walking through water can result in the release of soluble gases that would not normally be in the mine atmosphere.

a.  True

b.  False

- The degree to which a toxic gas will affect you depends on:

a.  How concentrated the gas is

b.  How toxic (poisonous) the gas is

c.  How long you’re exposed to the gas

d.  All of the above

e.  None of the above.

- If you wear your self-contained breathing apparatus in petroleum based fumes for prolonged or successive periods, the fumes can eventually permeate its rubber parts so that the apparatus no longer provides you with adequate protection.

a.  True

b.  False

- Oxygen deficiency in a mine can result from:

a.  Insufficient ventilation

b.  Displacement by other gases

c.  Fire or explosion

d.  Worker consumption

e.  All of the above.

- Exposure to low levels of carbon monoxide (0.15 to 0.20 percent) is not dangerous.

a.  True

b.  False

- High nitrogen dioxide readings could indicate:

a.  There has been a fire or explosion

b.  Malfunctioning electrical equipment producing arcs     or sparks

c.  Inadequate ventilation of diesel equipment

d.  All of the above

e.  None of the above

- Which of the following applies to hydrogen sulfide?

a.  It can be flammable and explosive in the right         concentration

b.  It is one of the most poisonous gases known

c.  It is colorless but tastes like rotten eggs

d.  All of the above

e.  Only a. and b. above

- Methane will be found only in bituminous coal mines or near   oil/gas wells that penetrate metal/nonmetal mines.

a.  True

b.  False

- Blackdamp is a mixture of carbon dioxide, nitrogen, air and   probably carbon monoxide and can cause suffocation.

a.  True

b.  False

- During rescue and recovery operations, the main fan(s)        should be monitored or guarded to insure that it operates    continuously.

a.  True

b.  False

- Map legends and symbols are uniform throughout the mining     industry.

a.  True

b.  False

- A properly installed regulator will not affect the quantity   of airflow.

a.  True

b.  False

- Mine rescue teams should close bulkhead man doors as soon as   they are found open and then immediately notify the command   center.

a.  True

b.  False

- Being able to determine the direction and velocity of         airflow enables the team to check whether the ventilation    system is functioning as it should in a given area.

a.  True

b.  False

- Bare-faced exploration may be conducted when:

a.  The ventilation system is operating properly

b.  There is sufficient oxygen and no buildup of           dangerous gases

c.  A backup crew with apparatus is immediately            available

d.  All of the above

- Sometimes “runners” are stationed at the fresh air base to    carry messages to the rescue team during exploration work.

a.  True

b.  False

- The duties of the fresh air base coordinator are:

a.  Handle communication with the team and the command     center

b.  Map the team’s progress and findings

c.  Coordinate and oversee activities at the fresh air     base

d.  All of the above

- The first priority in exploration is to rescue survivors.

a.  True

b.  False

- It is recommended that you make your first stop for a team    check as soon as possible after leaving the fresh air        base.

a.  True

b.  False

- If locomotives are used for exploration, it is recommended    that at least two be used in case one breaks down.

a.  True

b.  False

- The principle that underlies almost all firefighting methods   is the removal of one leg of the fire triangle (fuel,        oxygen, heat).

a.  True

b.  False

- The National Fire Protection Association classifies fires     into five classes: A, B, C, D, & E.

a.  True

b.  False

- Direct firefighting involves certain hazards to the team,     including:

a.  Electrocution

b.  Toxic and asphyxiating gases

c.  Oxygen deficiency and explosive gases

d.  Heat, smoke, and steam

e.  All of the above

26.	In the code of federal regulations 49.1 “availability of mine rescue teams” clearly outlines the requirements of a mine rescue team except teams operating under 49.3 or 49.4

a. true

b. false

27.	After an explosion has occurred, the hazards a team might encounter are:

a. spreading fires

b. further explosions

c. weakened ground conditions

d. all of the above

28.	The preferred type of extinguisher for teams is a dry chemical type that contains:

a. sodium bicarbonate

b. potassium chloride

c. carbon tetrachloride

d. monoammonium phosphate

29.	The appropriate rate of compressions during adult CPR is

(    ) per minute.

a. 80 to 100

b. no faster then 80

c. about 100

d. no faster then 120

30.	Infectious diseases range from such generally mild conditions as the common cold to life-threatening diseases such as tuberculosis. The four diseases of most concern the emergency medical responders are:

a. human immunodeficiency virus, hepatitis, tuberculosis, meningitis

b.	AIDS, hepatitis, blood borne pathogen, influenza

c. typhoid fever, viral encephalitis, bacterial meningitis, SARS

d. hepatitis, tuberculosis, meningitis, Lyme disease

31.	two ways to best minimize the stress associated with responding to emergencies are to work closely with other more experienced responders and:

a. stay out of harms way

b. medication

c. practice your skills often

d. don’t recognize stress as being a problem

32.	An elevated concentration of carbon dioxide (CO2) in a mine can be harmful because:

a. it is highly explosive

b. it increases the breathing rate

c. it is highly toxic in small concentrations

d. all of the above

33.	During an emergency MSHA will issue a 103A order. This order is intended to protect miners and prevents the company from doing anything without MSHA oversight.

a. true

b. false

34.	A MSHA District mamanger can revoke a mine rescue instructor’s approval to instruct for good cause. Once decided by the District manager this is a final decision.

a. true

b. false

35.	A mine rescue team member will be ineligible to serve on a team if (    ) hours are missed during a one year period.

a. 4 hours

b. 6 hours

c. 8 hours

d. 10 hours