2010 National Metal and Nonmetal Mine Rescue Contest

Mine Rescue Field Competition – Written Test

Please do not write on this test.  Use the answer sheet provided.

- Rock gas is largely _______________.

  - Nitrogen and carbon dioxide

  - Nitrogen and carbon monoxide

  - Carbon dioxide and hydrogen sulfide

  - Hydrogen and carbon monoxide

  - None of the above

- Information the team relays to the fresh air base as it proceeds is known as the “progress report”.

  - True

  - False

- Fires that involve flammable or combustible liquids are:

  - Class A

  - Class B

  - Class C

  - Class D

  - None of the above

- An information center should only be directed by a company official.

  - True

  - False

- The explosive range of Ethane in normal air is ______________ percent.

  - 1.86 to 8.41

  - 5 to 15

  - 3 to 12.5

  - 2.12 to 9.35

  - None of the above

- In order to get air to flow from the intake to the exhaust, the exhaust air must be at a higher pressure than the intake.

  - True

  - False

- ____________ is found in high concentrations in black powder smoke.

  - Whitedamp

  - Afterdamp

  - Blackdamp

  - Firedamp

  - None of the above

- When conducting barefaced exploration, a back-up crew with apparatus should be stationed outside the area.

  - True

  - False

- Any testing device containing electrical circuits used in mines classified as gassy must be tested by MSHA prior to use under such conditions.

  - True

  - False

- ____________ has a slight garlic odor.

  - Butane

  - Ethane

  - Methane

  - Acetylene

  - None of the above.

- The location of a victims cap lamp should be marked on the mine map and on the side of the mine near where they are found.

  - True

  - False

- A ____________ is used to measure high velocity airflow in ducts or tubing.

  - Smoke tube

  - Pitot tube

  - Medium velocity anemometer

  - High velocity anemometer

  - None of the above

- During a mine emergency, _____________ should establish the command center and oversee all aspects of the rescue and recovery operation.

  - Top MSHA official

  - Top State official

  - Mine manager/Superintendent

  - CEO of company

  - None of the above

- It is easier for concentrations of explosive gases to build up when the barometric pressure is high.

  - True

  - False

- The duties of the Fresh Air Base Coordinator typically include

  - Checking names or numbers of the team going into the affected area.

  - Checking condition of backup team.

  - Checking and logging equipment and materials.

  - All of the above

  - None of the above

- The characteristics of the burning material and the surrounding strata are one of the main factors in determining when to unseal a fire area.

  - True

  - False

- ______________ is a mixture of carbon monoxide, carbon dioxide, methane, oxygen, nitrogen, and hydrogen.

  - Whitedamp

  - Stinkdamp

  - Afterdamp

  - Blackdamp

  - Firedamp

- It is okay to allow a survivor to walk out on their own if they appear to be in good shape.

  - True

  - False

- It is the duty of the ____________ to notify all persons on the notification plan and inform them of the emergency.

  - Manager/Superintendent

  - Mine Clerk

  - Outside Supervisor

  - Safety Director

  - None of the above

- It is up to the team captain to decide whether or not to alter the ventilation during exploration of the mine.

  - True

  - False

- The explosive range of carbon monoxide in normal air is __________ percent.

  - 4 to 74.2

  - 12.5 to 74.2

  - 4.3 to 45.5

  - 5 to 15

  - None of the above

- A decrease in atmospheric pressure causes gas to expand.

  - True

  - False

- If the team is stopped at one location for an extended period, it’s common practice for the fresh air base to signal two short pulls on the communication line about every 5 minutes to check on the team’s condition.

  - True

  - False

- If a body is in cold water, it will remain in almost perfect condition for:

  - 4 – 5 days

  - 7 – 8 days

  - 2 – 3 days

  - A week

  - None of the above

- Chemical analysis of air samples can reveal the presence of gases that portable detectors are not designed to detect.

  - True

  - False

- Usually, after temporary seals are erected, a waiting period of about 48 hours is recommended before beginning construction on permanent seals.

  - True

  - False

- ____________ is a hazard of direct firefighting.

  - Electrocution

  - Oxygen deficiency

  - Steam

  - All of the above

  - None of the above

- The most accurate method of determining air velocity with a smoke tube is to take readings:

  - In the center of the airway

  - At half points within the airway

  - Along the side of the airway

  - At quarter points within the airway

  - None of the above

- ________________ is a first priority condition in the triage system.

  - Back injury

  - Fractured arm

  - Multiple lacerations

  - Chest injury

  - None of the above

- ____________________ ventilation is the re-ventilation of a sealed area in successive blocks by means of air locks.

  - Block

  - Direct

  - In-Direct

  - Progressive

  - None of the above