Carlsbad New Mexico

Written Field Test

2010

- Under normal conditions the only two gases that will be found at an underground mine is Oxygen and nitrogen.

A. true

B. false

- The explosive range for Carbon Monoxide in normal air is

  - 5.5 to 10.5%

  - 12.5 to 50%

  - 12.5 to 74.5%

  - 15.5 to 75%

- Sulfur Dioxide gas concentrations as low as ______ is dangerous to life

  - .02 to .07%

  - .01 t0 .05%

  - .04 to .05%

  - .4 to .5%

  - .004 to .005%

- The chemical symbol for acetylene is

  - C2H4

  - C3H6

  - C2H2

  - C4H4

  - A2C4

- When a team goes into a mine to explore or fight a fire, it should be concerned with two main things.

  - Is there a back up team and are they trained

  - Spreading of the fire and is there a second way out

  - Spreading of the fire and the possibility of an explosion

  - Possibility of an explosion and poor ventilation

  - Is the team properly trained for this mission and do they have all of the equipment they will need.

    - Before going underground, the team should make sure that the main fan is running, that a guard is monitoring the operation of the fan and

a. The command center knows where you are going

- A back up team is able and ready to enter the mine

- Test are being made at the main exhausts for any gases that may be present

- The command center will not let anyone else in the mine until your team says it’s OK.

- A gas is a substance with no definite shape but does have a definite volume.

    - A gas is a substance with no definite shape but does have a definite volume.

- true

- false

    - Hydrogen sulfide is most explosive at 14.2%.

- true

- false

    - Which one of the following gases has an explosive range of 4.5 to 45.3%?

- hydrogen sulfide

- butane

- acetylene

- none of the above

    - Methane has an explosive range of about 5 to 15%. For methane to burn or explode there must be at least ____ % oxygen present.

- 12.1

- 15.2

- 17.2

- 9.2

    - The explosive range of ethane is _____in normal air.

- 3 to 9.35%

- 2.12 to 12.5%

- 1.86 to 8.41%

- None of the above

    - White-damp is a mixture of carbon dioxide and air which results from a mine fire.

- true

- false

    - To obtain the flow of air through a mine there must be a difference in air velocity between the intakes and exhaust airways.

- true

- false

    - In order to get air to flow from the intake to the exhaust, the exhaust air must be ____ pressure than the intake.

- constant

- equal

- higher

- lower

    - Map legends and symbols are not uniform throughout the mining industry.

- true

- false

    - Velocity is the rate of airflow in square feet per minute.

- true

- false

    - A temporary bulkhead should be erected at least  ___ to ___ feet into a passage way to allow a permanent bulkhead to be constructed at a later date.

- 8 to 10

- 10 to 12

- 6 to 8

- 4 to 6

    - Team checks should be conducted to ___________.

- make sure each team member is fit and ready to continue

- make sure each team member’s apparatus is functioning properly

- to give the team a chance to rest

- all of the above

    - Monitoring the mine atmosphere for the presence of oxygen, explosive gasses, and carbon monoxide is another important element in team ____.

- communication

- exploration

- development

- progression

    - When a team goes into a mine to explore for a fire or to fight a fire, it should be concerned with ____ main thing(s).

- one

- two

- three

- four

    - Re-ventilation after an explosion in a single-level, room and pillar mine is usually accomplished by progressive ventilation.

- true

- false

    - The main objectives of exploration work during a mine fire are;

- extinguishing the fire and assessing conditions

- checking for gases and extinguishing the fire

- assessing conditions and locating the fire

- none of the above

    - The best way of relieving psychological stress in survivors is to trey to _____ them as soon as possible.

- rescue

- advance fresh air to

- communicate with

- sit with

    - The human body will decay very rapidly at temperatures above ____ degrees.

- 60

- 65

- 70

- 50

    - When approaching a fire you should stay low to avoid any roll back of flames and try to get within 6 to 8 feet of the fire before turning on the fire extinguisher.

- true

- false

    - If the team finds more then one survivor in a refuge chamber the team should:

- Do a triage and decide who should leave first, provide breathing protection and send the person to fresh air via shortest route.

- Do a triage and decide who should go first, leave a team member with the remaining person to keep him/her calm and ensure them the team will be returning.

- Do a triage and load the first selected person onto a stretcher, provide breathing protection and transport to the nearest fresh air base, repeat the process until all survivors are rescued.

- Provide a w-65 self rescuer to all of the survivors and get them to fresh air as soon and as quick as you can.

    - If a body is found the first thing a team member should do is check all of his/her pockets and look for something that will identify the miner.

- true

- false

    - When measuring ventilation with a smoke tube one must convert FPS to FMP. Therefore if the smoke travels 23 feet in 60 seconds it would convert to:

- 28 fpm

- 38 fpm

- 48 fpm

- None of the above

    - The best way to prevent hydrogen pops during a fire fighting event is to:

- build a fire regulator

- decrease air flow

- provide adequate air flow over the fire

- None of the above

    - If you suspect rubber, neoprene and PVC is burning in a mine one gas that may be present is arsine and its maximum allowable part per million would be

- 0.5

- 0.05

- 0.005

- 5.0