2010 Central Mine Rescue Contest

May 14-15, 2010
Kellogg, Idaho

Written Exam for the Mine Rescue Field Event

All tests must be completed within a total of 60 minutes

Mine/Organization___________________________________________________________

Name & Position #_______________________________________________

1.   Decreased atmospheric pressure has an effect on the diffusion rate of a gas by slowing it down.

a.	true

b.  false

2.   The TLV for carbon monoxide is 50 ppm or:

a.  0.5%

b.  0.05%

c.  0.005%

d.  none of the above

3.   Smoke is a result of combustion.  It consists of tiny particles of ____________ and __________ matter suspended in air.

a.  semi-solid and low density

b.  toxic and combustible

c.  solid and liquid

d.  all of the above

4.   Afterdamp is toxic, explosive, and can be oxygen deficient.  It contains the following mixture of gases:

a.  methane, propane, pentane, sub-micron particulates, and air

b.  carbon dioxide, sulfur dioxide, nitrogen, argon, and air

c.  carbon monoxide, carbon dioxide, methane, oxygen, nitrogen, and hydrogen

d.  carbon monoxide, oxygen, water vapor,  oxides of nitrogen, and air

5.   Very tiny amounts of sulfur dioxide ( ______% or less) will irritate your eyes and respiratory tract.

a.  0.0001

b.  0.001

c.  0.01

d.  none of the above

6.   The purpose of a(n) ____________ is to separate two different atmospheres while still permitting miners to enter and exit without mixing the atmospheres.

a.  regulator

b.  overcast

c.  airlock

d.  bulkhead

7.   The pitot tube used for measuring air flow in ducts or tubing has a U-tube water gauge or some other differential pressure gauge for determining the velocity pressure inside the duct or tubing.

a.  true

b.  false

- 8.   High expansion foam removes two legs of the fire triangle.  They are:

a.  fuel and oxygen

b.  oxygen and heat

c.  heat and fuel

d.  none of the above

- 9.   Chest injuries take priority over a fractured arm under the triage system.

a.  true

b.  false

- 10. Moderate shock takes priority over a fractured arm under the triage system.

a.  true

b.  false

- 11. A regular anemometer actually measures ________________ of the air.

a.  velocity

b.  gas concentrations

c.  linear feet of travel

d.  area

- 12. Of the following gases, which one is not soluble in water?

a.  methane

b.  sulfur dioxide

c.  hydrogen

d.  none of the above

13. It is recommended that you do not drink alcoholic beverages for at least ______ hours before you go under oxygen.

a.  6 to 12

b.  10 to 15

c.  12 to 18

d.  none of the above

14. A high velocity anemometer is used for measuring velocities from ______________ feet per minute.

a.  2,000 to 12,000

b.  1,000 to 10,000

c.  2,000 to 10,000

d.  1,000 to 12,000

15. Oxygen is:

a.  lighter than air.

b.  heavier than air.

c.  less dense than air.

d.  equal in density to air.

16. Using a smoke tube, you determine the air velocity is 100 feet per minute through a drift that is 10 feet high by 10 feet wide.  What is the quantity of airflow through this drift in cubic feet per minute?

a.  1,000 ft3/min

b.  10,000 ft3/min

c.  20,000 ft3/min

d.  200,000 ft3/min

e.  None of the above

17. Increased temperature can cause mine gases to do what?

a.  expand and diffuse more quickly

b.  contract and diffuse less quickly

c.  contract and diffuse more quickly

d.  expand and diffuse less quickly

18. In the “Fire Triangle,” a combustion chain reaction is not possible if one of the legs of the “Fire Triangle” is missing.

a.  true

b.  false

19. Oxides of nitrogen can be produced by sparking or electrical arcing.

a.  true

b.  false

20. Natural ventilation occurs due to:

a.  external wind

b.  differences in air pressure caused by temperature differences between surface and underground

c.  mining equipment moving through mine workings

d.  both b & c

21. A smoke tube test is used when the air velocity is less than:

a.  600 feet per minute

b.  120 feet per minute

c.  900 feet per minute

d.  180 feet per minute

22. Class B fires are fires that involve:

a.  metals like magnesium

b.  electrical fires

c.  flammable or combustible liquids

d.  ordinary combustibles like paper or wood

23. A ________ extinguisher would be appropriate for extinguishing a fire involving plastic or rubber.

a.  Class A

b.  Class B

c.  Class C

d.  both a and b

24. MSHA requires mine rescue stations serving underground M/NM mines to have how many gas detectors for each gas which may be encountered at the mine they serve?

a.  two

b.  three

c.  four

d.  five

25. A mine rescue communication line must have the tensile strength sufficient for it to be used as a manual communication system.

a.  true

b.  false

26. Your team is not going to be rescuing survivors, but will be erecting ventilation controls. Do they need a spare apparatus with them underground?

a.  yes

b.  no

27. Teams must have gas detection capability to measure the content of oxygen from 0.0 percent to at least 20.0 percent?

a.  true

b.  false

28. The ________ is used to link team members together.

a.  com line

b.  safety line

c.  linkline

d.  both a and c

29. A smoke tube is used to measure what?

a.  air velocity

b.  air direction

c.  air quantity

d.  all of the above

e.  both a and b

30. Ethane, propane, and butane are _____________.

a.  heavy hydrocarbons

b.  toxic and explosive gases

c.  colorless and tasteless

d.  usually found near the mine roof

e.  all of the above

f.   both a and c

2010 Central Idaho Mine Rescue Contest

May 14-15, 2010
Kellogg, Idaho

Answer Key for Written Exam for the Mine Rescue Field Event

All questions and answers taken from the 2008 edition of MSHA Publication 3027 (formerly IG 6)

Question	       Answer	          Page

1.		b		2-6

2.		c		2-9

3.		c		2-26

4.		c		2-27

5.		b		2-21

6.		c		3-23

7.		a		3-16

8.		b		5-10

9.		a		6-5

10.		a		6-6

11.		c		3-16

12.		c		2-19

13.		c		4-12

14.		c		3-16

15.		b		2-51

16.		b		3-20

17.		a		2-5 and 2-6

18.		a		5-4

19.		a		2-18

20.		b		3-5

21.		b		3-18

22.		c		5-50

23.		a		5-5

24.		c		2-3

25.		a		4-13

26.		b		4-14

27.		a		2-3

28.		c		4-53

29.		e		3-16

30.		f		2-23