New Iberia

Team trainer Test

2010

- Module 4 of the Metal Non Metal revised 2008 edition of “A guide to Mine Rescue Training” is

  - Mine gases

  - Mine ventilation

  - Exploration

  - Mine Recovery

  - None of the above

- Module 5 in the guide to mine rescue training discusses

  - Mine recovery

  - Rescue of survivors

  - Fires

  - Mine ventilation

  - None of the above

- 30 CFR, part 49.1 discusses

  - Availability of mine rescue teams

  - Alternative mine rescue capabilities for small mines

  - Purpose and scope

  - Definitions

- 30 CFR,  part 49, subpart B is for Metal Non Metal teams

  - True

  - False

- 30 CFR, 49.3 an operator can apply for small and remote status if there are fewer then ______ underground employees within  two hours of ground travel time,

  - 25

  - 35

  - 26

  - 36

  - 15

- 49.7 under physical requirements for mine rescue team members a team member must have a repeated blood pressure reading that does not exceed

  - 150 over 100

  - 160 over 120

  - 160 over 100

  - 150 over 90

  - None of the above

- Mine rescue equipment repair records shall be kept for

  - 1 year

  - 2 year

  - No requirement

  - Until reviewed by an Authorized representative from MSHA

- Which of the following is not true of sulfur dioxide?

  - It is explosive

  - It is highly toxic

  - It is highly soluble in water

  - It can occur during mine fires

- An elevated concentration of carbon dioxide in mine air can be harmful because;

  - It is highly explosive

  - It increases the breathing rate

  - It is highly toxic in small concentrations

  - All of the above

- Characteristics of hydrogen sulfide include:

  - Explosive

  - High toxic

  - Can be liberated from pools of stagnant water

  - All of the above

- Gases that are neither toxic nor explosive:

  - Are not found in mine atmospheres

  - Are not dangerous

  - Can be dangerous because they can displace oxygen

  - Cannot be detected with today’s detection instruments

- Carbon dioxide (CO2) seeks high places

  - True

  - False

- Nitrogen Dioxide (NO2) seeks high places

  - True

  - False

- Hydrogen (H2) seeks high places

  - True

  - False

- Sulfur Dioxide (SO2) has a specific gravity of 2.2638, therefore should be tested for in low areas. It has a very low explosive range and is very toxic.

  - True

  - False

- Infectious diseases range from such generally mild conditions as the common cold to life-threatening diseases such as tuberculosis. The four diseases of most concern to emergency medical responders are:

  - Human immunodeficiency virus, hepatitis, tuberculosis, meningitis.

  - AIDS, hepatitis, blood borne pathogen, influenza

  - Typhoid fever, viral encephalitis, bacterial meningitis, SARS

  - Hepatitis, tuberculosis, meningitis, Lyme disease

- The appropriate rate of compressions during adult CPR is _____per minute.

  - 80 to 100

  - No faster then 80

  - About 100

  - No faster then 120

- Pulmonary resuscitation is:

  - Any effort to restart normal heart rhythms.

  - Any effort to revive or restore normal breathing.

  - The use of mechanical devices to restart breathing.

  - The ability to restore normal heart rhythm and breathing.

- Mine rescue teams should alter existing ventilation:

  - Only when directed to do so by the command center

  - When the team captain decides to do so

  - When they encounter high concentrations of carbon monoxide

  - When they encounter smoke

- Burning materials that give off extremely toxic gases in addition to carbon monoxide:

  - Timbers

  - Hydraulic fluids

  - Neoprene and other synthetic rubber compounds

  - All of the above

- Barefaced exploration should be attempted only when:

  - The team captain says it OK

  - Everyone on the team says there up for it

  - A back up mine rescue team with apparatus is immediately available

  - There are no indications an apparatus is needed.

- In the Code of federal regulations 49.2 “Availability of mine rescue teams” clearly outlines the requirements of a mine rescue team except teams operating under 49.3 or 49.4

  - True

  - False

- It is much easier to remove a concentration of sulfur dioxide by ventilation than it is to remove the same concentration of hydrogen

  - True

  - False

- Which of the following is not a heavy hydrocarbon?

  - Ethane

  - Propane

  - Butane

  - Acetylene

- Sulfur Dioxide gas in concentrations as low as _____ is dangerous to life.

  - .02 to .07 %

  - .01 to .05 %

  - .04 to .05 %

  - .4 to .5 %

  - .004 to .005 %

- If you took a ventilation reading in a drift 14 feet wide and 16 feet high and record a true reading of 1200 fpm, what is the CFM?

  - 2400

  - 250,00

  - 268,000

  - 286,800

- Taking a reading in a drift 10 feet wide and 10 feet high and a true reading of 1000 feet per minute the CFM would be.

  - 1000

  - 10,000

  - 100,000

  - None of the above

- If a survivor is found in contaminated or questionable air and is fully conscious and ready to travel, the team should

  - Get them to a fresh air base as soon as possible

  - Let them continue to head for a fresh air base so the team can do more important work

  - Provide the person with breathing protection and send them toward fresh air.

  - None of the above

- In body recovery, a muscular individual will begin putrefaction sooner then an obese person.

  - True

  - False

- If a body is found the first thing a team member should do is check all of his/her pockets and look for stuff that will identify him/her.

  - True

  - False

- The fact that Nitrogen Dioxide (NO2) is non toxic when encountered by a team they should map and mark it but move on and worry about more dangerous gases.

  - True

  - False

32.   When measuring ventilation with a smoke tube one must convert FPS to FPM. Therefore 23 seconds

60 seconds= ___________

a. .28 minute

b. .38 minute

c. .48 minute

d. none of the above

- Medium velocity anemometer is used to determine air velocities from ________to _______fpm.

  - 100/1000

  - 120/1200

  - 120/2000

  - 1000/3000

- Team signals of one= stop, two signals= advance, three signals= retreat, four signals= distress are international signals all team members know and use.

  - True

  - False

- The best way to prevent hydrogen pops during a fire fighting event is to

  - Build a fire regulator

  - Decrease air flow

  - Provide adequate air flow over the fire

  - None of the above