TECHNICIAN TEAM WRITTEN TEST

BG4 Breathing Apparatus and Draeger X-am 2000 Gas Instrument

Winnemucca, Nevada - 2010

Name: ___________________________       Company: _______________________

Team Name: _____________________        Contest Position Number: ________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

BG4 Breathing Apparatus:

1. What are the types of BG4 testing?

A. Quick electronic operational check

B. Portable test kits

C. Dynamic bench tester

D. All above

2. The BG4 has been flame tested to NIOSH open circuit standards?

A. True

B. False

3. The high pressure transducer converts pressure into a digital signal?

A. True

B. False

4. The Panarama Nova mask has a double sealing surface?

A. True

B. False

5. The lenses for the mask can be plexiglass?

A. True

B. False

6. The panorama Nova provides 90% peripheral vision?

A. True

B. False

7. The over pressure valve opens < 2 mbar?

A. True

B. False

8. Using a refillable canister, the concerns are: the lid o-ring and the tab

Lock connectors?

A. True

B. False

9. The panorama Nova mask can use a harness or hairnet?

A. True

B. False

10. The minimum valve is installed within the breathing loop?

A. True

B. False

MSHA Publication 3027 Modules 2 and 3:

11. A gas with a specific gravity of 0.8 will seek high places.

A. True

B. False

12. Oxygen deficiency with an oxygen concentration of 9% will result in

unconsciousness.

A. True

B.  False

13. Temporary bulkheads can not be constructed by fitting brattice around a

piece of equipment.

A. True

B.  False

14. Carbon monoxide has an explosive range of 12.5 to 74.2 percent in normal air.

A. True

B.  False

15. As little as 2,000 ppm of hydrogen sulfide can cause immediate death.

A. True

B.  False

16. A medium-velocity anemometer is used to measure air velocities from _____

to 2,000 feet per minute?

- 50

- 100

- 120

- 150

17. To obtain the “flow” of air through a mine there must be a difference in both air velocity and pressure between the intake and exhaust airways.

A. True

B.  False

18. The condition of the following should be reported to the command center.

- Ventilation controls

- Compressed air lines

- Auxiliary fans

- All of the above

19. A smoke tube is used to show only the direction of slow-moving air.

- True

- False

20. How many of the following statements are true?

- Carbon dioxide will burn.

- Carbon dioxide is insoluble in water.

- Carbon dioxide is produce by decaying of timbers.

- Breathing air containing 5 percent carbon dioxide increases respiration 300 percent.

- 1

- 2

- 3

- 4

Draeger X-am 2000 Gas Instrument:

21.  The Cat Ex (methane/combustible) sensor may show incorrect results in an oxygen-poor atmosphere.

A. True

B. False

22. Dirt and deposits can be removed from the device by washing it with cold water.

A. True

B. False

23. The manual states "Not tested in oxygen enriched atmospheres."

A. True

B. False

24. The gas instruments can accept the following battery types _________.

A.  NiMHy

B.   zinc-carbon

C.  alkaline

D. all of the above

E.  both A and C

25. In the case of (pre-alarm) A1, a double tone is audible and the alarm LED

flashed twice.

A. True

B. False

26. After the leaving the area, if the concentration is less than the alarm setpoint A2.

Press the [OK] key, the alarm messages are switched off.

A. True

B. False

27. Switching/turning the Draeger X-am 2000 on by pressing and holding the [OK] key for approx. ____________ seconds ….

A. 1

B.  3

C.  5

D. 7

E. 9

28.  Benzene can damage the CAT Ex (methane/combustible) sensor.

A. True

B. False

29.  The manufacturer recommends before entering the workplace that a bump test must be performed ...

A. True

B. False

30.  Under Battery pre-alarm, the manual states "The battery still lasts approx. 20 minutes after the first battery pre-alarm."

A. True

B. False