TECHNICIAN TEAM WRITTEN TEST

BG4 Breathing Apparatus and Industrial Scientific Gas Instruments

Winnemucca, Nevada - 2010

Name: ___________________________       Company: _______________________

Team Name: _____________________        Contest Position Number: ________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

BG4 Breathing Apparatus:

1. What are the types of BG4 testing?

A. Quick electronic operational check

B. Portable test kits

C. Dynamic bench tester

D. All above

2. The BG4 has been flame tested to NIOSH open circuit standards?

A. True

B. False

3. The high pressure transducer converts pressure into a digital signal?

A. True

B. False

4. The Panarama Nova mask has a double sealing surface?

A. True

B. False

5. The lenses for the mask can be plexiglass?

A. True

B. False

6. The panorama Nova provides 90% peripheral vision?

A. True

B. False

7. The over pressure valve opens < 2 mbar?

A. True

B. False

8. Using a refillable canister, the concerns are: the lid o-ring and the tab

Lock connectors?

A. True

B. False

9. The panorama Nova mask can use a harness or hairnet?

A. True

B. False

10. The minimum valve is installed within the breathing loop?

A. True

B. False

MSHA Publication 3027 Modules 2 and 3:

11. A gas with a specific gravity of 0.8 will seek high places.

A. True

B. False

12. Oxygen deficiency with an oxygen concentration of 9% will result in

unconsciousness.

A. True

B.  False

13. Temporary bulkheads can not be constructed by fitting brattice around a

piece of equipment.

A. True

B.  False

14. Carbon monoxide has an explosive range of 12.5 to 74.2 percent in normal air.

A. True

B.  False

15. As little as 2,000 ppm of hydrogen sulfide can cause immediate death.

A. True

B.  False

16. A medium-velocity anemometer is used to measure air velocities from _____

to 2,000 feet per minute?

- 50

- 100

- 120

- 150

17. To obtain the “flow” of air through a mine there must be a difference in both air velocity and pressure between the intake and exhaust airways.

A. True

B.  False

18. The condition of the following should be reported to the command center.

- Ventilation controls

- Compressed air lines

- Auxiliary fans

- All of the above

19. A smoke tube is used to show only the direction of slow-moving air.

- True

- False

20. How many of the following statements are true?

- Carbon dioxide will burn.

- Carbon dioxide is insoluble in water.

- Carbon dioxide is produce by decaying of timbers.

- Breathing air containing 5 percent carbon dioxide increases respiration 300 percent.

- 1

- 2

- 3

- 4

Gas Instruments:

21.  If a sudden change in atmospheric pressure occurs when travelling through mine ventilation doors or ascending a shaft, it may cause temporary fluctuations in the oxygen reading.

A. True

B. False

22. If the instrument water barriers are not kept clean, obstructions may cause readings to be lower than actual gas concentrations.

A. True

B. False

23. Oxygen enriched atmospheres may cause combustible gas reading to be higher than actual concentrations.

A. True

B. False

24. The gas instruments can accept the following battery types _________________.

(circle only one answer under the options for your gas instrument)

For the TMX412:			                       For the iTX and MX6 iBrid:

A.  nickel-cadmium (NiCad) 		       A. lithium-ion

B.   zinc-carbon				                 B.  zinc-carbon

C.  alkaline					                      C.  alkaline

D. all of the above				            D. all of the above

E.  both A and C				                 E.  both A and C

25. In latching mode for the gas instrument, all high alarm conditions (and not in

Over Range condition) will automatically clear/turn off when the gas

concentration has decreased to a level below the high alarm setpoint.

A. True

B. False

26. Verify the calibration of the combustible gas sensor after any incident where the

combustible gas content has caused the instrument to latch or lock in the OVER-

RANGE (alarm) conditions.

A. True

B. False

27. If the sensor's full span value or span reserve is less than ______________ of the applied (calibration) gas value\concentration, the sensor will fail calibration.

A. 30%

B. 50%

C. 70%

D. 90%

E. 100%

28.  Benzene can affect the combustible gas sensor and may cause readings of

combustible gas to be lower than actual gas concentrations.

A. True

B. False

29.  The manufacturer recommends that a functional (“bump”) test be performed

on every instrument prior to each days use.

A. True

B. False

30.  When the battery life is nearing its end, the following occurs.

(answer the question for your gas instrument)

| For the TMX412, when approximately 1 hour of energy remains, gas instrument emits a short beep every 15 seconds. In addition, the battery indicator will be replaced by a flashing “B” on the display | For the iTX, with a minimum of 15 minutes of battery life, the unit will emit a periodic tone. | For the MX6 iBrid, if the remaining runtime is less than 10 minutes, “Low Battery” is displayed. |
| --- | --- | --- |

A. True

B. False