2010 National Metal and Nonmetal Mine Rescue Contest

Technician Team Competition (BioPak 240R & S) – Written Test

Please do not write on this test.  Use the answer sheet provided.

10 Questions for multi-gas instrument (TMX412, iTX, and MX6 iBrid):

- Which of the following may cause the combustible gas sensor to read lower than actual gas concentrations?

A. Silicone compound vapors

B. Oxygen deficient atmospheres

C. Sudden changes in atmospheric pressure

D. all of the above

E. only A and B

- A functional (bump) test is defined as a brief exposure of the (gas) monitor to a known concentration of gas(es) for the purpose of verifying sensor and alarm operation and to measure the accuracy of the instrument.

A. True

B. False

- High Over-Range (+OR or OR) or off-scale combustible gas reading may indicate an explosive concentration of combustible gas.

A. True

B. False

- When the gas instrument is in non-latching mode, alarms set according to the Technician Team Competition in Contest Rule Book and exposed to 19.9% Oxygen, 0.9% Methane, 40.2 ppm Carbon Monoxide and 25.0 ppm Nitrogen Dioxide, it will ______________.

A. be in high alarm condition

B. display "40" for the Carbon Monoxide reading

C. display "25.0" for the Nitrogen Dioxide reading

D. all of the above

E. only A and C

- Marginal calibration occurs if the (gas) sensor's full span value or span reserve is between 50% – 70% of the applied (calibration) gas value\concentration.

A. True

B. False

- For a low alarm condition (for the MX6 iBrid the audio and visual alarms are enabled), the gas instrument will activate only an audible alarm and the corresponding gas display or value will flash.  For a high alarm, a visual alarm will also be activated.

A. True

B. False

- The gas instrument indicate a "low battery warning or alarm" on the display by ______ (select answer under the options for your gas instrument)

For TMX412:		     For iTX			     For MX6 iBrid

A. a flashing "B" 		A. display "Battery Low	A. display "Low Battery"

Starting Shutdown"	     in red

B. an empty battery 	B. a flashing "B" 		B. a flashing "B"

indicator

C. flashing "LOW BAT" 	C. flashing "LOW BAT"	C. flashing "LOW BAT"

D. none of the above	D. none of the above 		D. none of the above

- When does the oxygen (O2) sensor receive its calibration for the gas instrument?

A. During the countdown timer

B. After zeroing is complete for the toxic and combustible sensors

C. During the "Sensor Configuration Gas" mode.

D. During the "Hygiene Function" mode.

- During calibration of the gas instrument, what is the recommended flow rate?

A. 0.25 liter per minute (Lpm)

B. 0.5 liter per minute (Lpm)

C. 1.5 liter per minute (Lpm)

D. 2.0 liter per minute (Lpm)

- For the gas instrument, the maximum display range for the (catalytic) combustible\methane\LEL sensor is ______________

- 5.0% Methane

- 100% LEL

- 50% LEL

- Both A and B

- Both A and C

10 Questions from MSHA Publication 3027 - Modules 2 and 3:

- From the team’s findings on the conditions of ventilation controls, the team will issue       directions to the command center on how to reestablish ventilation in the mine.

A. True

B.  False

- An anemometer actually measures linear feet of travel.

A. True

B. False

- The most accurate method of determining the air velocity using a smoke tube is to take readings at quarter points within the airway?

A. True

B. False

- Air containing 5 to 74.2 percent hydrogen will explode even when there is as little as             4 percent oxygen present?

A. True

B. False

- Heat from a fire in the mine will cause gases to expand and ___________?

A. be harder to disperse.

B. be dispersed more easily.

C. be diffuse more slowly.

D. both A and C

- Gases issuing into still air without mixing tend to stratify according to the gas's specific gravity?

A. True

B.  False

- Which of the following is not true for oxides of nitrogen, such as Nitrogen Dioxide and Nitric Oxide?

A. will neither burn nor explode

B. are highly toxic

C. heavier than air

D. reddish-brown at low concentrations and dark brown at higher

concentrations

- Carbon dioxide is the most poisonous of the gases in afterdamp?

A. True

B.  False

- How many of the following statements are true?

  - Carbon monoxide (CO) is flammable.

  - Carbon monoxide is slightly soluble in water.

  - Exposure to as little as 0.15 to 0.20 percent CO is extremely dangerous.

  - As little as 500 ppm carbon monoxide can kill you in three hours.

- 1

- 2

- 3

- 4

- Changes in pressure and temperature will have the following effect on a gas ______________.

A. an increase in pressure causes a gas to contract

B. a decrease in temperature causes a gas to expand

C. both A and B

D. none of the above

10 Questions from Biomarine BioPak 240R & S – User and Benchman Manuals and Quizzes:

- In a properly working positive pressure NIOSH/MSHA approved Closed Circuit breathing apparatus the pressure inside the breathing loop/circuit is?

- Lower than ambient or negative pressure

- The same as ambient, neutral

- The same as cylinder pressure

- Slightly higher than ambient or positive

- If the user exhales and the breathing diaphragm/breathing chamber fills to a specific capacity, the breathing chamber will cause the activation of the?

A. By-Pass Valve

B. Demand Valve

C. Relief/Vent Valve

23. The purpose of the CO2 absorbent is to remove Carbon Monoxide from the breathing gasses?

A. True

B. False

- What best describes the breathing circuit/loop in a BioPak 240 R or S apparatus?

A. It’s the route that exhaled gas takes as it exits the mask and travels through the breathing chamber, scrubber and the cooler then exiting to the atmosphere

B. It’s the route that exhaled gas takes as it exits the mask and travels through the breathing chamber, the scrubber and the cooler and reenters the mask.

- If Positive Pressure could be attributed to only one component in the BioPak what would it be?

A. The breathing chamber

B.  The oxygen pressure

C. The regulator

D. The springs

- The function of the pressure regulator is to?

A. Increase the cylinder pressure

B. To control moisture

C. Reduce the oxygen cylinder pressure

D. A&C

- The Check valves in the mask/hose adapter perform a critical function; which of the following is not true?

A. They control the directional flow of inhalation and exhalation gases

B. The inhalation valve is closed during exhalation

C. The exhalation valve is closed during inhalation

D. They control the amount of O2 being metered out from the O2 cylinder

- When the CO2 absorbent is used up; the material now becomes?

A. Lithium Hydroxide

B. Lithium Carbonate

C. Calcium Hydroxide

D. Calcium Carbonate

- A small leak in the breathing circuit/loop will cause the relief/vent valve to open?

A. True

B. False

- One of the probable causes for high breathing resistance during exhalation is the demand valve in the breathing chamber has failed.

A. True

B. False