TECHNICIAN TEAM WRITTEN TEST

Biomarine Breathing Apparatus and

Industrial Scientific Corporation Gas Instruments

Winnemucca, Nevada - 2010

Name: ___________________________       Company: _______________________

Team Name: _____________________        Contest Position Number: ________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

Biomarine Breathing Apparatus:

1. MSHA requires that Periodic Maintenance be performed at least ____ on Mine Rescue Breathing Apparatus.

- Daily

- Weekly

- Every 30 days

- Quarterly

2. Carbon Fiber wrapped Oxygen cylinders must be hydrostatically tested every _________ .

- One Year

- Three Years

- Five Years

- Fifteen Years

3. In a properly working positive pressure NIOSH/MSHA approved Closed Circuit breathing apparatus the pressure inside the breathing loop/circuit is:

- Lower than ambient

- The same as ambient

- The same as cylinder pressure

- Slightly higher than ambient

4. If a slight leak occurs in the breathing loop/circuit or facemask seal, the user generally will _________:

- Hit the demand valve more often

- Reduce duration

- Increase oxygen consumption

- All of the Above

- None of the Above

5. Turn-around Maintenance should be performed _________ .

- As soon as possible after each use

- On a monthly basis

- During periodic maintenance

- During long-term maintenance

6. There are various methods to eliminate or reduce mask fogging.  Which one is the least likely to work

:

- Depress the Emergency By-Pass Valve

- Use an approved wiper

- Use an approved anti-fog spray or solution

- B & C

7. When completing washing/disinfection during turn-around maintenance users may use any antibacterial cleaner they choose because _________ .

- Bleach works on any germ so I can use it

- Alcohol is used in hospitals so it works on my apparatus

- Any liquid detergent is OK since it won’t hurt my skin

- None of the above

8. What will BEST result in the best cooling of breathing gasses:

- Install the gel tube/ice canister just prior to donning

- Install the gel tube/ice canister during benching back at the rescue station

- Do not use the gel tube/ice canister at all.

- Use only partially frozen gel tube/ice canister

9. During rescue operations in oxygen deficient and high CO atmosphere a team member’s apparatus goes into low Oxygen alarm.  Which best describes your actions:

- Do nothing, press on with the rescue.

- Tell the member to breath slower

- Tell the member to go back to the fresh air base alone.

- Each team member checks their oxygen supply and the team retreats to correct the problem.

- Have him remove his mask and change out the oxygen cylinder

10. If the constant add or the demand valve fails in your apparatus, the user can still manually fill the breathing chamber by activating the

- Emergency By-Pass Valve

- Demand Valve

- Relief Valve

- Check Valve

MSHA Publication 3027 Modules 2 and 3:

11. A gas with a specific gravity of 0.8 will seek high places.

A. True

B. False

12. Oxygen deficiency with an oxygen concentration of 9% will result in

unconsciousness.

A. True

B.  False

13. Temporary bulkheads can not be constructed by fitting brattice around a

piece of equipment.

A. True

B.  False

14. Carbon monoxide has an explosive range of 12.5 to 74.2 percent in normal air.

A. True

B.  False

15. As little as 2,000 ppm of hydrogen sulfide can cause immediate death.

A. True

B.  False

16. A medium-velocity anemometer is used to measure air velocities from _____

to 2,000 feet per minute?

- 50

- 100

- 120

- 150

17. To obtain the “flow” of air through a mine there must be a difference in both air velocity and pressure between the intake and exhaust airways.

A. True

B.  False

18. The condition of the following should be reported to the command center.

- Ventilation controls

- Compressed air lines

- Auxiliary fans

- All of the above

19. A smoke tube is used to show only the direction of slow-moving air.

- True

- False

20. How many of the following statements are true?

- Carbon dioxide will burn.

- Carbon dioxide is insoluble in water.

- Carbon dioxide is produce by decaying of timbers.

- Breathing air containing 5 percent carbon dioxide increases respiration 300 percent.

- 1

- 2

- 3

- 4

Industrial Scientific Corporation Gas Instruments:

21. If a sudden change in atmospheric pressure occurs when travelling through mine ventilation doors or ascending a shaft, it may cause temporary fluctuations in the oxygen reading.

A. True

B. False

22. If the instrument water barriers are not kept clean, obstructions may cause readings to be lower than actual gas concentrations.

A. True

B. False

23. Oxygen enriched atmospheres may cause combustible gas reading to be higher than actual concentrations.

A. True

B. False

24. The gas instruments can accept the following battery types __________.

(circle only one answer under the options for your gas instrument)

For the TMX412:			                       For the iTX and MX6 iBrid:

A.  nickel-cadmium (NiCad) 		       A. lithium-ion

B.   zinc-carbon				                 B.  zinc-carbon

C.  alkaline					                      C.  alkaline

D. all of the above				            D. all of the above

E.  both A and C				                 E.  both A and C

25. In latching mode for the gas instrument, all high alarm conditions (and not in

Over Range condition) will automatically clear/turn off when the gas

concentration has decreased to a level below the high alarm setpoint.

A. True

B. False

26. Verify the calibration of the combustible gas sensor after any incident where the

combustible gas content has caused the instrument to latch or lock in the OVER-

RANGE (alarm) conditions.

A. True

B. False

27. If the sensor's full span value or span reserve is less than ______________ of the applied (calibration) gas value\concentration, the sensor will fail calibration.

A. 30%

B. 50%

C. 70%

D. 90%

E. 100%

28.  Benzene can affect the combustible gas sensor and may cause readings of

combustible gas to be lower than actual gas concentrations.

A. True

B. False

29.  The manufacturer recommends that a functional (“bump”) test be performed

on every instrument prior to each days use.

A. True

B. False

30.  When the battery life is nearing its end, the following occurs.

(answer the question for your gas instrument)

| For the TMX412, when approximately 1 hour of energy remains, gas instrument emits a short beep every 15 seconds. In addition, the battery indicator will be replaced by a flashing “B” on the display | For the iTX, with a minimum of 15 minutes of battery life, the unit will emit a periodic tone. | For the MX6 iBrid, if the remaining runtime is less than 10 minutes, “Low Battery” is displayed. |
| --- | --- | --- |

A. True

B. False