TECHNICIAN TEAM WORKSHOP

April 7, 2010, Carlsbad, New Mexico

TECHNICIAN TEAM WRITTEN TEST

Name: ___________________________       Company: _______________________

Team Name: _____________________        Contest Position Number: ________

Directions: Circle the letter preceding the correct answer to each of the following questions.  Circle only one answer per question.

Gas Instruments:

1.  If a sudden change in atmospheric pressure occurs when travelling through mine ventilation doors or ascending a shaft, it may cause temporary fluctuations in the oxygen reading.

A. True

B. False

2. If the instrument water barriers are not kept clean, obstructions may cause readings to be higher than actual gas concentrations.

A. True

B. False

3. The calibration of the combustible gas sensor should be verified after any incident where the combustible gas content has caused the instrument to display an over range condition.

A. True

B. False

4. The gas instruments can accept the following battery types _________________.

For the TMX412:			     For the iTX and MX6 iBrid:

A.  nickel-cadmium (NiCad) 		A. lithium-ion

B.   zinc-carbon				B.  zinc-carbon

C.  alkaline					C.  alkaline

D. all of the above				D. all of the above

E.  both A and C				E.  both A and C

5.  The calibration is marginal if the reserve span is ________ the calibration gas.

A. greater than

B. 30%-50% of

C. 50%-70% of

D. 70%-90% of

E. lower than

6. For a standard calibration, the gas instrument will display for each sensor the amount of sensor life left in percent.

A. True

B. False

7. If the sensor's full span value or span reserve is less than ______________ of the applied (calibration) gas value\concentration, the sensor will fail calibration.

A. 30%

B. 50%

C. 70%

D. 90%

E. 100%

8.  Silica can affect the combustible gas sensor and may cause readings of

combustible gas to be lower than actual gas concentrations.

A. True

B. False

9.  The manufacturer recommends that a functional (“bump”) test be performed

on every instrument prior to each days use.

A. True

B. False

10.  When the battery life is nearing its end, the following occurs.

TMX412				iTX				MX6

| the instrument emits a shortbeep every 15 seconds. In addition, the battery indicator will be replaced by a flashing “B” on the display | The unit will emit a periodic tone. | “Low Battery” is displayed. |
| --- | --- | --- |

A. True

B. False

11. What are the types of BG4 testing ?                                                            	 _____

A. Quick electronic operational check	B. Portable test kits		Pg 64		C. Dynamic bench tester			D. All above

12. The Bg4 has been flame tested to Niosh  open circuit standards.               	______

Pressure gauge.								Pg.61

A. True	B.False

13. The high pressure transducer converts pressure into a digital signal ?		______

A. True	B. False							pg.61

14. The Panarama Nova mask has a double sealing surface ?				______

A. True	B.False							pg 61

15. The lenses for the mask can be plexiglass ?					______

A. True	B. False							pg 60

16. The panorama Nova provides 90% peripheral vision ?				______

A. True		B. False							pg 60

17. The over pressure valve opens < 2 mbar ?

A. True	B. False							pg 57

18. Using a refillable canister, the concerns are : the lid o-ring and the tab		______

Lock connectors ?									pg 56

A. True	B False

19. The panorama Nova mask can use a harness or hairnet ?				______

A. True	B. False							pg 60

20. The minimum valve is installed within the breathing loop ?			______

A. True	B. false							pg 53

MSHA Publication 3027 Modules 2 and 3:

21. A gas with a specific gravity of 1.2 will seek high places.  False MODULE 2-41

- True

- False

22. Oxygen deficiency with an oxygen concentration of 10% will result in unconsciousness.  False MODULE 2-51

- True

- False

23. Temporary bulkheads can be constructed by fitting brattice around a piece of equipment.  True MODULE 3-22

- True

- False

24. Propane has a specific gravity of 1.5625 and an explosive range of 2.12 to 9.35 percent.  True MODULE 2-47

- True

- False

25. Oxygen deficiency with an oxygen concentration of 5% will almost always result in death.  True MODULE 2-14

- True

- False

26. A medium- velocity (or “regular”) anemometer is used to measure air velocities from 100 to 2,000 feet per minute?  B MODULE 3-16

- True

- False

27. A high-velocity anemometer is for measuring velocities from _____ to 10,000

feet per minute?  C MODULE 3-16

- 1000

- 1500

- 2000

- 2500

28. The condition of the following should be reported to the command center.  D MODULE 3-15

- Ventilation controls

- Compressed air lines

- Auxiliary fans

- All of the above

29. Non-sparking shovels should be used to fill in around temporary bulkheads.  True MODULE 3-22

- True

- False

30. How many of the following statements are true?.  B MODULE 2-5

- An increase in atmospheric pressure causes a gas to expand.

- A decrease in atmospheric pressure causes a gas to contract.

- An increase in temperature causes a gas to expand.

- A decrease in temperature causes a gas to contract.

- 1

- 2

- 3

- 4