2010 Central Mine Rescue Contest

May 14-15, 2010
Kellogg, Idaho

Technician Team Written Exam

All tests must be completed within a total of 60 minutes

Name: ___________________________       Company: _______________________

Team Name: _____________________        Contest Position Number: ________

Directions: Write the correct letter on the blank space provided at the right.

BG4 Breathing Apparatus:

1. What are the types of BG4 testing?                                     			______

A. Quick electronic operational check

B. Portable test kits

C. Dynamic bench tester

D. All above

2. The temperature for drying parts must not exceed 60 degress C.			______

A. True	B. False

3. Both the blue and yellow lines are high pressure from the reducer?		______

A. True	B. False

4. One of the causes of incorrect dosage can be cylinder pressure is incorrect?	______

A. True	B. False

5. To check the exhalation valve, the Rz must be set on negative pressure?		______

A. True	B. False

6. Aviator grade oxygen can be used to fill the cylinder?				______

A. True	B. False

7. Permissable impurities in depressurized gas: Max. water vapor 			______

concentration: 50 mg/m3

A. True	B. False

8. Max drying temperature is ___ degrees F .						______

A. 120	   B 114    C 140	D. 130

9. A leak in the dosage line can cause high dosage 					______

A. true 	B. False

10. When the last low pressure warning occurs,_____ % of oxygen has been	______

used up.

A. 25	B. 75	C. 95	D. 5

iTX Gas Instrument:

11. Oxygen enriched atmospheres may cause readings of combustible                      _____

(methane) gas to be ______________ than actual concentrations.

A.  higher

B.  the same

C.  lower

12. Silicone compound vapors may cause readings of combustible gas                    _____

to be lower than actual gas concentrations.

A. True 	B. False

13. Solvents or cleaning solvents can be used to clean the outside                           _____

of the iTX instrument.

A. True	B. False

14. With a minimum of ____________ of battery life remaining, the iTX unit        _____

will emit a periodic tone alerting the user to charge/replace the battery.

A. 5 minutes

B. 15 minutes

C. 30 minutes

D. 1 hour

15. Under the specifications of the manual, the iTX will have a run time                  _____

of _________________ under the following conditions:

a fully charged Li-ion pack; all sensors installed; room temperature;

no alarms activated; and without a parasitic pump.

A. 10 hours

B. 15 hours

C. 20 hours

D. 24 hours

16. For calibration, any gas sensor with a full span value greater than                      _____

_________ of the calibration gas value will show PASS.

A. 30%

B. 50%

C. 70%

D. 100%

17. Sudden changes in atmospheric pressure may cause temporary                          _____

fluctuations in the ______________ reading.

A. carbon monoxide

B. methane

C. nitrogen dioxide

D. oxygen

E. all of the above

18. A functional (bump) test is defined as ______ .                                                  ______

A. a brief exposure of the monitor (gas instrument) to a known

concentration of gas (es)

B. to verify sensor operation

C. to verify alarm operation

D. all of the above

E. none of the above

19. Verify the calibration of the combustible gas sensor after any                            _____

incident where the combustible gas content has caused the instrument

to latch in the OVER-RANGE (alarm) conditions.

A. True	B. False

20. The oxygen sensor will calibrate to room (clean) air at _______                        _____

A. 20.0%

B. 20.5%

C. 20.9%

D. 23.0%

MSHA Publication 3027 Modules 2 and 3:

21.  The presence of small quantities of hydrogen (H2)                                           ______

has little effect on the explosive range of other gases.

A. True      B.  False

22. Exposure to an atmosphere containing 15% oxygen                                            _____

will cause dizziness and headaches.

A. True      B. False

23. Which of the following gases is not toxic?                                                           _____

A. Oxides of Nitrogen

B. Carbon Dioxide

C. Acetylene

D. All of the above

24. The specific gravity of Nitrogen Dioxide (NO2) is ______ .                               _____

A. 1.7493

B. 2.0756

C. 1.5894

D. 2.9153

25. An anemometer actually measures ________________ of air.                            _____

A. linear feet of travel

B. quantity

C. velocity

D. all of the above

Metal and Nonmetal Mine Rescue Contest

Kellogg, Idaho - 2010

ANSWERS for the TECHNICIAN TEAM WRITTEN TEST

BG4 Breathing Apparatus:

- D		pg 64

- B		pg 45

- B		pg 44

- A		pg 41

- A		pg 36

- A		pg 28

- A		pg 28

- C		pg 27

- B		pg 28

- C		pg 13

iTX Gas Instruments:

- A		Manual (rev 6), page 3

- A		Manual (rev 6), page 3

- B		Manual (rev 6), page 27

- B		Manual (rev 6), page 27

- D		Manual (rev 6), page 29

- C		Manual (rev 6), page 25

- D		Manual (rev 6), page 3

- D		Manual (rev 6), page 23

- A		Manual (rev 6), page 3

- C		iTX on-line Calibration Training Video, Slide 8

MSHA Publication 3027 Modules 2 and 3:

- B		Module page 2-18

- A		Module page 2-14

- B		Module page 2-28

- C		Module page 2-17

25        A		Module page 3-16