Southern Regional Mine Rescue Contest

New Iberia, LA

May 6-8, 2014

Written Test – Field Competition

Name____________________________	Company___________________________

Team Name_______________________	Contest Position No.__________________

Team Member No._________________

Directions: Use answer sheet & fill in completely.

- During a rescue and recovery operation, the only duty of the outside Supervisor is to set up an area where food and drinks are available for the rescue teams and other personnel.

  - True

  - False

- The explosive range of Ethane in normal air is?

  - 2.12 – 9.35%

  - 3.0 – 12.5%

  - 2.5 – 9.35%

  - None of the above

- Reestablishing ventilation and bringing fresh air to an area damaged by fire or explosion is the main task of mine rescue teams in a rescue operation.

  - True

  - False

- The law requires that gas detectors be able to measure concentrations of methane from 0.0 to 100% of volume, oxygen from 0.0 to 20% of volume, and carbon monoxide from 0.0 to 9,999 ppm.

  - True

  - False

- Air locking operations should never be undertaken until the oxygen content of the air behind the seals has been reduced to at least _________%.

  - 2.0%

  - .05%

  - 1%

  - 5.0%

- Urethane foam should never be applied greater than 1 ½ inch thick because of spontaneous combustion.

  - True

  - False

- You can allow a rescued miner to walk out of the mine alone if the miner appears to be in good shape?

  - True

  - False

- Usually, after temporary seals are erected, a waiting period of about 72 hours is recommended before beginning construction on permanent seals?

  - True

  - False

- When constructing a temporary bulkhead from a crosscut, the bulkhead should be ____ feet into the passageway?

  - 10 to 12

  - 1 to 2

  - 4 to 6

  - None of the above.

- The smoke tube is a device that emits a smoke cloud which floats along with the air current to show the quantity of the airflow and the approximate velocity of the airflow.

  - True

  - False

- If you are exposed to high concentrations of CO, you may experience very few symptoms before losing consciousness.

  - True

  - False

- Carbon Dioxide is colorless and odorless, but high concentrations may produce an acid taste.

  - True

  - False

- ____________ is found after methane explosions in air having low oxygen content?

  - Butane

  - Ethane

  - Acetylene

  - Argon

- The debriefing session is a very important aspect of your team’s body recovery operation.

  - True

  - False

- Heavier gases such as carbon dioxide and sulfur dioxide do not diffuse rapidly, so they’re easier to disperse?

  - True

  - False

- The degree to which a toxic gas will affect you depends on three factors: 1) how concentrated the gas is 2) how explosive the gas is and 3) how long you’re exposed to the gas?

  - True

  - False

- _________ are spring-loaded expandable metal rods that can be used to erect a temporary stopping.

  - Curtain rods

  - Post sticks

  - Stopping rods

  - Pogo sticks

- Oxides of nitrogen are highly toxic. Breathing even small amounts will cause irritable bowel syndrome.

  - True

  - False

- A line brattice is brattice cloth or plastic that is hung to channel intake air into a working area that otherwise would not have adequate ventilation.

  - True

  - False

- The purpose of a permanent stopping is to separate two different atmospheres while still permitting miners to enter and exit without mixing the atmospheres.

  - True

  - False

- The team briefing is normally conducted:

  - Only at a mine rescue contest

  - At the fresh air base

  - Command Center

  - None of the above

- Rock –Strata gases commonly called “rock gas” is assumed to contain what two types of gasses?

  - Hydrogen and Carbon Monoxide

  - Nitrogen and Sulfur Dioxide

  - Nitrogen and Carbon Dioxide

  - Hydrogen Sulfide and Hydrogen

- Normally gasses stratify when the ventilation system in a mine is working properly.

  - True

  - False

- In a multi-level mine, a tunnel driven perpendicular to the main vein system of the mine is considered a?

  - Drift

  - Stope

  - Crosscut

  - None of the above

- Another source of nitrogen in underground mines is the detonation of explosives.

  - True

  - False

- Before using indirect ventilation, there should be conclusive evidence that the fire has been extinguished.

  - True

  - False

- The advantage of progressive ventilation is it is a fast process.

  - True

  - False

- Bulkheads are used to direct air to where it is needed and to keep intake air from short-circuiting to the exhaust before it reaches the working area.

  - True

  - False

- A fresh air base is usually established at a point where conditions no longer permit barefaced exploration.

  - True

  - False

- Check curtains are designed to close automatically after you have passed through them so that they continue to direct air to the working place.

  - True

  - False