Southwest Wyoming Mutual Aid Mine Rescue Contest

Rock Springs WY

June 10-12, 2014

DRAEGER BG4 Team Technician Test Answers

1. b) MSHA requires mine rescue stations serving underground M/NM mines to have four gas detectors appropriate for each gas which may be encountered at the mines served. MSHA Publication 3027, Page 2-3

2. a) In addition, gas detectors must measure concentrations of methane from 0.0 percent to 100 percent of volume, oxygen from 0.0 percent to at least 20 percent of volume, and carbon monoxide from 0.0 parts per million to at least 9,999 parts per million. MSHA Publication 3027, Page  2-3

3. b) The volume of a gas changes in response to any change in atmospheric pressure or temperature. For example: An increase in temperature causes a gas to expand. MSHA Publication 3027, Page 2-5

4. c) The specific gravity of normal air is 1.0

MSHA Publication 3027, Page 2-6

5. b) Sulfur dioxide, for example, has a specific gravity of 2.2638. MSHA Publication 3027, Page 2-6

6. d)  The explosive range of hydrogen, for example, is 4.0 to 74.2 percent in the presence of normal air.  MSHA Publication 3027, Page 2-7

7. b) In order to get the air to flow from the intake to the exhaust, the exhaust air must be at a lower pressure than the intake. MSHA Publication 3027, Page 3-5

8. a) Bulkheads are used to direct air to where it is needed and to keep intake air from short circuiting to the exhaust before it reaches the working area.  MSHA Publication 3027, Page 3-8

9. a) Check curtains are used to deflect the intake air current into a working

area. MSHA Publication 3027, Page 3-9

10. b) Any teams working beyond the established time period will be notified by the #1 Judge that they must leave the field. 2014 Metal/Non Metal Mine Rescue Rules Page 7

11. d) Charging the Oxygen Cylinder, Permissible impurities in depressurized gas: Max. water vapor concentration: 50 mg/m3.  PSS BG 4 Service Manual Page 17

12. c)  Fitting the Oxygen Cylinder The cylinder must be full for operational usage, 3000 PSI (206 BAR) is required.  PSS BG 4 Service Manual Page 20

13. b) Testing the PSS BG 4 with Inserted CO2 Absorber  Exceptionally, if the breathing circuit is opened for more than 5 minutes, the CO2 absorber apertures

should be sealed with three PE plugs to prevent escape of gas.  PSS BG 4 Service Manual Page 23

14. a) Low Pressure Warning Indication – Backlight turns on including red LED

PSS BG 4 Service Manual Page 24

15. d) Checking the Exhalation Valve 1 Set “negative pressure pumping”. 2 Tightly pinch the inhalation hose with your hand. 3 Pump once until -10  mbar is indicated on the pressure gauge. PSS BG 4 Service Manual Page 24

16. d) Leak Test with Positive Pressure 1 Set “Leak test” 2 Press the button to reduce the pressure to 7 mbar. Start the stopwatch. The pressure must not change by more than 1 mbar during one minute. PSS BG 4 Service Manual Page 26

17. b) . High-Pressure Leak Test 1 Set “Leak test”. 2 Open the cylinder valve. 3 Watch the Sentinel: the cylinder pressure is indicated here. If cylinder pressure is lower than 2600 psi / 180 bar:  Charge the oxygen cylinder. PSS BG 4 Service Manual Page 28

18. b) When Checking the Minimum Valve Watch the pressure gauge:

the minimum valve should open at a value between 0.1 and 2.5 mbar.

PSS BG 4 Service Manual Page 29

19. c) Checking the Residual Pressure Warning • Close the cylinder valve. 1 Watch the Sentinel. A warning should be generated at approximately 700 psi / 48 bar. The acoustic alarm sounds intermittently and the red LEDs flash.

PSS BG 4 Service Manual  Page 31

20. a) Symbol “Battery warning 2”– In this case the red LEDs flash constantly. On

Power up 5 short beeps will sound and the display will show the “Battery warning 2” icon. Sentinel automatically powers off and blocks any further operation.

PSS BG 4 Service Manual Page 32

21. b) If a battery warning 2 is displayed, the battery must be replaced before any new operation.   PSS BG 4 Service Manual Page 32

22. d)  Switching Off the Sentinel • Simultaneously press the right and left hand

button until a single sharp audible “bleep”sounds.• Release buttons.• For 3 seconds Sentinel shows the battery status.• Sentinel switches off.  PSS BG 4 Service Manual Page 33

23. c) The O ring on plug –in coupling should be replaced yearly when in constant use. PSS BG 4 Service Manual Page 38

24. b) The Major Component Groups for the BG4 are Pneumatics, Breathing loop, Sentinel System, and Case & Harness.  PSS BG 4 Service Manual Page 43

25. c) BG 4 High Pressure O2 Cylinders• Steel cylinder is hydrotested at 5 year intervals  PSS BG 4 Service Manual Page 44

26. a) BG 4 Minimum Valve• Activates between .1 and 2.5 mbar  PSS BG 4 Service Manual Page 46

27. d) BG 4 Minimum Valve • Provides approximately 80 l/min flow

PSS BG 4 Service Manual Page 46

28. b) BG 4 Breathing Hoses • EPDM hose material • Quarter turn connectors• Single style hose• Anti-crush rings   PSS BG 4 Service Manual Page 47

29. d) BG 4 Relief Valves• Two sealing valves installed Moisture trap valve Over pressure relief • Components identical • Over pressure valve opens between 2-5 mbar • Moisture trap opens ≥ 15 mbar   PSS BG 4 Service Manual Page 50

30. b) . BG 4 Breathing Bag • 5.5 Liter volume• Sealed seams • Polyurethane / yellow PSS BG 4 Service Manual Page 51